A New Method for the Calculation of Hydrophilic Surface Area (HSA) from Structure

James A. Quinn,

Norgwyn Montgomery Software Inc.

216 Lower Valley Rd.,

North Wales PA 19454 USA

e-mail: rsajaq@erols.com

The interaction of molecules with water is a driving force in many chemical and biological processes.  Many existing methods of calculating interactions of molecules with water and other solvents exist and are used in trying to understand these processes.  These calculated interactions include water solubility 12, 17, 21, HLB (hydrophilic-lipophilic balance) 3,5,6,18, log octanol-water partition coefficient  7, 8, 15, 20, polar surface area 4, the solvochromatic parameters 10,11, the 3-D solubility parameters 1,9,19, energy of cohesion 1, 19 and mean water of hydration 18.  In this paper a new method for calculating hydrophilic surface area (HSA) is presented that is fairly simple and robust and can be used in place of many of the calculation methods listed above.  It should be useful to synthesis chemists in the rational design of new molecules.

Contents

1.  Calculation of hydrophilic surface area

2.  Comparison of HSA with other measures of hydrophilicity

2.1 Comparison of hydrophilic surface area and HLB (hydrophilic-lipophilic balance)

2.2 Comparison of HSA with polar surface area (PSA)

2.3 Comparison of HSA with the Log of the Octanol Water Partition Coefficient (Log P)

2.4 Comparison of HSA with the Hildebrand solubility parameter

2.5 Comparison of HSA with water solubility

3.  Use of HSA in graphic displays

4.  Example of use of the concept in design of new molecules

5.  Tables of Hydrophilic Surface Area values

Table 2-1. Stimulants, anti-depressants, anti-psychotics and nootropic drugs

Table 2-2. Pesticides

Table 2-3.  Solvents

Table 2-4.  Surfactants

6. Conclusions

1. Calculation of hydrophilic surface area

The method consists of three steps and a final summation.  First, calculate the surface area of each atom.  Second, look-up the relative hydrophilicity of the atom from a table, and third, calculate the influence of neighboring atoms on hydrophilicity of the atom.  If an atom is hydrophilic then add its surface area to the summed hydrophilic surface area.  Percent hydrophilic surface area is easily computed by dividing the summed hydrophilic surface area by the total surface area.

I use the program Molecular Modeling Pro to calculate the surface area, hydrophilic surface area and percent hydrophilic surface area.  This program calculates surface area analytically (with geometry).  Since 3-D structure affects the result, geometry optimization is a necessary first step.  With Molecular Modeling Pro this means either running a MOPAC geometry minimization, or conformational analysis with an MM2 minimization.  The program then calculates surface area using the van der Waals radii from a paper by Bondi2 and the following equations:

Surface area of a sphere = 4r2

The area of overlap between atoms is subtracted from the surface area for each atom.

Overlap area between two atoms = *2*r1*(r1-(dist12/2+(r12-r22)/(2*dist12)))

Where r1 and r2 are the van der Waals radii of atoms 1 and 2, and dist12 is the distance between them.

From these two equation the exposed van der Waals surface area of each atom can approximated.

The next step is to give each atom an initial value for hydrophilicity.  Values of pi from Hansch and Leo's 1979 compendium7 (mainly from the table on page 23) were used to derive the initial atomic values for organic molecules.  Some additional values were added to account for anionic and cationic materials based on HLB values of surfactants.  A partial list of the initial values is listed in Table 1.

| Table 1. initial hydrophobic values by atom type |  |
| --- | --- |
| Atom type | Initial hydrophilic value |
| Hydrogen | 0.23 |
| Non-aromatic C | 0.2 (add 0.69 for CH3) |
| Aromatic CH | 0.13 |
| Aromatic CR | 0.22 |
| =O | -2.1 |
| Charged O and OH in acids | -5.19 |
| Aromatic OH | -0.8 |
| Non-aromatic OH | -2.1 |
| Ether oxygen, not next to aromatic | -1.82 |
| Ether oxygen, next to one aromatic ring | -0.61 |
| Ether oxygen, between two aromatic rings | 0.53 |
| Charged nitrogen | -4.8 - (2*formal charge) |
| Triple bonded nitrogen | -1.47 |
| Nitrogen bonded to oxygen | -1.5 + 1.24*each attached oxygen |
| Nitrogen in aromatic ring (=N-) | -1.12 |
| Nitrogen in aromatic ring (-NR-) | -1.6 |
| Nitrogen in aromatic ring (-NH-) | -0.85 |
| Single bonded nitrogen attached to aromatic ring | -0.93 |
| Nitrogen between two aromatic rings | -0.32 |
| Double bonded nitrogen attached to aromatic ring | -1.46 |
| Non-aromatic nitrogen, RN(R)R | -2.18 (subtract 0.46 for NH2) |
| Non-aromatic nitrogen, R=NR | -1.6 |
| Aromatic fluorine | 0.37 |
| Non-aromatic fluorine | -0.38 |
| Aromatic chlorine | 0.96 |
| Non-aromatic chlorine | 0.06 |
| Aromatic bromine | 1.09 |
| Non-aromatic bromine | 0.2 |
| Aromatic iodine | 1.35 |
| Non-aromatic iodine | 0.59 |
| Phosphorus (RP(=R)(R)R) | -1.94 + 0.8*number of oxygens |
| Sulfur, in a ring that is mostly or all aromatic | 0.36 |
| Sulfur, not SCN, attached to aromatic ring | 0.03 |
| Sulfur, in SCN, attached to aromatic ring | 0.98 |
| Sulfur, non-aromatic | -0.79 |
| If sulfur has formal charge subtract from above | -2*(formal charge) |
| Metals - univalent | -7.8 |
| Manganese | -3.8 |

The initial hydrophilic values for each atom are then adjusted by the values for the other atoms in the molecule.  The adjustments are greater for neighboring atoms then for distant atoms (decrease geometrically by half).  The adjustment is done as follows.

Adjusted hydrophilicity for atom i =

initial hydrophilic value of atom i

+ 0.5 * the initial hydrophilic value of each adjacent atom j

+ 0.25 * the initial hydrophilic value of each atom k adjacent to the adjacent atoms j except for atom i

+0.125* the initial hydrophilic value of each atom m adjacent to atoms k except atom j

+0.0625*the initial hydrophilic value of each atom n adjacent to atoms m except atom k

+0.03125*the initial hydrophilic value of each atom o adjacent to atoms n except atom m

Reducing the contribution by half was decided upon empirically as this gives results consistent with how lipophilic and hydrophilic regions of surfactants are usually depicted.

Using the adjusted hydrophilicity value for each atom, the atoms are characterized as lipophilic (positive adjusted hydrophilicity) or hydrophilic (negative adjusted hydrophilicity)(i.e. 1 or 0).  The hydrophilic atoms' surface areas are summed and the result is the hydrophilic surface area (HSA) for the molecule (units for HSA from here on are cm2/mol *109).  It is this method of summing hydrophilicity, which does not add the actual adjusted hydrophilic value, but turns them into a one or zero that gives the method its robustness.  It implies that water either tends to interact with a region of the molecule or it doesn't - there is no in-between.  This behavior is encountered in nature in mean water of hydration.  This assumption may or may not be correct for a given use.

2. Comparison of HSA with other measure of hydrophilicity

2.1 Comparison of hydrophilic surface area and HLB (hydrophilic-lipophilic balance)

HLB, widely used by surfactant chemists as a measure of hydrophilicity, was originally defined by Griffin5, 6 as HLB = (E+P)/5 where E is the percentage weight of oxyethylene and P the percentage weight of polyhydric alcohol.  Alternatively he showed in the case of fatty acid esters of polyhydric alcohols that HLB can be calculated as HLB = 20*(1-saponification number/acid number).  In other words, HLB for nonionic materials is the percent of the molecule (by weight) which is hydrophilic divided by 5.  The relationship between HLB for nonionic compounds and percent HSA (100*HSA/Surface area) is obvious, given the similar way in which they or calculated (%HSA substitutes surface area for mass).  In theory, HLB should equal percent HSA* (molecular weight/surface area)/5 for nonionic materials.  Which is better to use depends on if molecular weight or surface area is the better descriptor of the chemical process being modeled.  For most chemical and biological processes this will be surface area.  The correlation between HLB and Percent HSA should be similar to the correlation between molecular weight and surface area and is depicted in figure 1a for 218 surfactants.  Since some anionic and cationic materials have been given HLB values above 20 (i.e. are not the percent weight that is hydrophilic divided by 5), this analogy breaks down for them. The relationship observed in figure 1a for compounds with HLB between 0 and 20  (including ionic molecules) can be expressed as:

HLB = 0.25  + 0.20 * percent HSA

r squared =  0.67, n = 218, probability model due to chance <0.00001

The correlation would be much better if only nonionic molecules and were included in the model:

HLB = -1.46 + 0.23 * percent HSA

poloxamer 101 left out as an outlier, r squared = 0.87, n=194, probability <0.00001

Clearly, some refinements to the way the HSA calculation handles anionic and cationic species may be warranted, although a better solution for reproducing HLB from HSA might be to add Free-Wilson type indicator variables for the ammonium, sodium and potassium ions and create a multiple regression equation.

HLB is often used by formulation and surfactant chemists as a guide to replacement of emulsifiers and detergents in a mixture, as compounds with similar HLBs often behave alike.  A question of the HSA and percent HSA calculations is if they can have wider use than this, given that they generate values for all types of compounds.

2.2 Comparison of HSA with polar surface area (PSA)

Polar surface area4 is conceptually similar to HSA and not surprisingly, the correlation between these calculations for compounds with PSA of 2000 or less is very strong (figure 1b).  The relationship for 859 gasses, solvents, polymers and surfactants was

Polar Surface Area =  23.2 + 2.95 * Hydrophilic Surface Area

r squared =  0.98, n = 859, probability << 0.00001

The relationship seems to break down somewhat for some polar polymers like polyacrylic acid or polyacrylamide (PSA > 2000) which would fit better in the data set described in the next paragraph.

Only a slightly worse correlation was found for a group of anti-depressants, anti-psychotics, stimulants and other neurologically active compounds.

Polar Surface Area = -14.4 +  7.56*Hydrophilic Surface Area

r squared= 0.96  n= 454  probability <<0.00001

However, the intercept and coefficient of regression were radically different from those obtained for the solvents, surfactants and polymers.  The compounds in this group contained many hydrogen bond donating groups and compounds of an ionic nature which appear to give this different relationship.  This reflects the origin of the initial HSA atom values in Hansch pi, which has similar values for an ether oxygen and a hydroxy  oxygen, whereas hydroxy oxygen has about double the PSA contribution compared to ether oxygen.  This neurological data set contained a few very large compounds which were well predicted and helped achieve the high r squared value.

PSA, although measuring much the same thing as the older HLB value has been put to quite different uses.  It has been used in place of, or in addition to the Log of the octanol-water partition coefficient in studies of biological transport and bioavailability in the pharmaceutical industry.  It appears that HSA and percent HSA could work equally well.  In fact HSA seems to be a variant calculation of PSA in many ways.

2.3 Comparison of HSA with the Log of the Octanol Water Partition Coefficient (Log P)

A single variable regression model combining surface area, molecular weight and percent HSA was found to correlate 80% of the variance in literature Log P values in a data set containing 228 solvents.

Log P = -0.37 + 0.20 * surface area *log10 (molecular weight/percent HSA)

R squared = 0.80, n = 220 solvents, model standard dev. = 0.77, F=1181, probability < 0.00001

This was among the stronger correlations with Percent HSA found.  However, when this correlation is extended to a list of over 300 pesticide Log P values, the correlation is much lower, with an r squared between 0.3 and 0.4.  Some curvature is seen in the relationship, and HSA is particularly poor at predicting very low Log P values, particularly of ionic materials or materials with many hydrogen bond donating groups.  The pesticide molecules are much more diverse than the solvent molecules, with everything from simple copper salts to fungicidal antibiotics. The solvent data set is from a paper by David Leahy13 where he looked at the relationship between Log P, Log water solubility and the solvochromatic parameters.

Lin and Marszall15 found a relationship between the partition coefficient and HLB:

HLB = 0.36 x ln(1/Kp) + 7

This relationship appears somewhat different than the one found with HSA above.

2.4 Comparison of HSA with the Hildebrand solubility parameter

The Hildebrand solubility parameter () has been correlated with HLB, so one would expect a similar relationships with HSA.  R.C. Little16 reported the following relationship between the solubility parameter and HLB:

HLB = 54*((/MPa1/2 - 16.8)/(/MPa1/2 - 12.3))

It would be tempting to substitute in the values from the previous equation relating HLB and percent hydrophilic surface area.  However, the correlation between both linear and non-linear models of the solubility parameter with HSA and Percent HSA was very poor.  This also applies to all of the Hansen 3-D solubility parameters.  These parameters are often used for predicting solubility or mixture affinity of compounds (like dissolves/associates in/with like).

2.5 Comparison of HSA with water solubility

Water solubility has proved to be a more difficult calculation from structure than the other measures of hydrophilicity discussed here.  This is because water solubility is also dependent on crystal lattice forces, which have a different underlying physical nature than the intrinsic hydrophilicity of the atom types.  Clearly correlating water solubility with any of the other calculated properties discussed here will fall short unless it can account for these forces.  An exception to this would be the correlation between these properties and the water solubility of liquids.  A nice relationship, for instance, was found by Leahy13, between Log P, Log water solubility and the solvochromatic parameters.  For Leahy's data set, the correlation between Log water solubility and surface area*log10(Molecular weight/HSA) was reasonable with an r squared of 0.62.  However this compares to an r squared of 0.83 for the combination of the solvochromatic pi star and cavity terms.

For data sets containing solids, melting point has been used to model the effect of the crystal lattice forces and reasonable correlations between Log water solubility and the combination of Log P and melting point have been found.  HSA is neither needed nor indicated to be a good underlying method for the calculation of water solubility, although, as expected a fair correlation was observed.  My hypothesis is that HSA is a measure of affinity for water, not solubility in water.

3. Use of HSA in graphic displays

The most straightforward use of HSA is in generated computer graphics.  HSA tends to divide molecules into hydrophilic and lipophilic regions.  This tends to be how many colloidal and surface chemists view molecules, and it will come as no surprise that the author of this paper was an industrial formulation chemist when he conceived the HSA calculation.

In figure 2, a graphical use of the calculation of hydrophilic surface area is shown.  The hydrophilic and lipophilic regions of the surfactant octoxynol-4 can be easily seen.  It shows how the calculation agrees with the common perception of surfactants being composed of regions of hydrophilicity and lipophilicity.  Such graphical representations were important in conceptualizing and validating the model.  For instance, the calculation correctly gives low percent HSA values when ethylene oxide is replaced by propylene oxide.  A glance at a picture like that in figure 2 will tell a chemist if a newly synthesized molecule is likely to have surfactant properties.   In figure 2 the values of adjusted HSA for the atom are used (not the 0 or 1 used in the HSA calculation).

4.   Example of Use of the concept in design of new molecules

The author of this paper worked for several years with synthetic chemists in developing QSAR models of biological activity to rationally direct discovery of new fungicide and insecticide molecules.  HSA was partially developed as an alternative predictor for inclusion in these models.  However, the most useful application of HSA may not be as numbers generated for developing structure-property relations.  The most useful application is probably in generating intuitive models from the graphical displays of the adjusted hydrophilicity of each atom.

As an example, consider the case of insecticides.  Using the graphical display of adjusted atomic hydrophilicity generated by Molecular Modeling Pro, it was noted that many of somewhat larger synthetic insecticides (with a wide range of biochemical modes of action) had a pattern of hydrophilicity.  This pattern can be described as a hydrophobic, conformationally flexible exterior, with a more rigid, hydrophilic core of about 0.5-0.8 nm in diameter, with a maximum molecular diameter no more than about 1.9 nm and a minimum molecular diameter of no more than 1 nm (figure 3).  If a molecule was more surfactant-like in character, that is a fairly linear molecule with one hydrophobic and one lipophilic region, then the maximum and minimum molecular diameter was less (figure 4).

If this view is correct, some simple rules for insecticide synthesis for molecules of the type shown in figure 3 (lipophilic exterior, hydrophilic core) follow:

- Molecules should not have a van der Waals maximum width perpendicular to the maximum diameter of greater than 1 nm.  The maximum diameter is usually not greater than 2 nm.

- For the hydrophilic core region, changing the hydrogen bonding characteristics or geometry will result in large swings in activity.  The easiest targets will not have changes in this region.

- The hydrophobic exterior can be changed in many ways, but should be left lipophilic, of limited diameter and conformationally flexible. The molecule should be able to assume a relatively lipophilic surface conformation when passing through the various biological filters, and another conformation at the biochemical site of action.  Adding substituents like NH2, OH, OSO3 or COOH to the exterior will result in a loss of ability to pass through the barriers to uptake.

- Hydrophilic requirements at the biochemical site of action may differ for the requirements of transport.  Compromise may be needed.

Insecticidal molecules that are larger than can be accommodated or that have hydrophilic exteriors should be suspected to be pro-drugs (break down in the gut to form a molecule that conforms to the model).

The biological rationale for this model is simple.  The organism must take in water soluble molecules up to 0.6 nm in minimum diameter, specifically sugars and amino acids (figure 5) as well as water itself.  It must not lose larger, water soluble molecules on which it has expended energy to create, such as enzymatic proteins and RNA.  Furthermore, the organism should be able to exploit lipophilic building blocks such as fatty acids while preventing excretion of larger, lipophilic molecules such as steroid hormones.  The insect has evolved a filtering mechanism that is described by the model for synthesis/transport and also by the size and hydrophilic characteristics of these natural molecules.

Examples of completely hydrophilic insecticides are difficult to find.  If one exists, it would be predicted to be no larger then simple sugars and amino acids.  Examples of completely lipophilic insecticides are well known: the chlorinated hydrocarbons.  For these molecules to fit the model, they must be able to be fit into the lipophilic exterior region of the proposed filtering model.

Correlations with Polar surface area and human intestinal absorption have been found to be sigmoidal for compounds with passive absorption4. The related HSA concept is likely to have a similar relationship and be applicable to correlating transport phenomenon outside of the insect example given here.

5.  Table of Hydrophilic Surface Area values

Table 2.1-2.4 contain the values of HSA and percent HSA for a large number of molecules, including all of those used in the comparisons of the various measures of hydrophilicity used in this study.

6. Conclusions

The calculation method of a new measure of hydrophilicity, hydrophilic surface area has been explained.  After examining its relationship to other measures of hydrophilicity, it appears that it can be used as an alternative to HLB (hydrophilic-lipophilic balance) and polar surface area.  It should be useful in applications as diverse as determining the interchangability of mixture components to transport of compounds across biological barriers.  It should be useful in helping synthesis chemists rationally design new compounds.  A computer program, Molecular Modeling Pro, has been developed which both calculates HSA and displays the adjusted atomic contributions graphically.

References:

- Barton, A.F.M., "Handbook of Solubility Parameters and other Cohesion Parameters", CRC Press, Boca Raton FL, 1983.

- Bondi, A. J. Phys. Chem., 1964,  68: 441-45.

- Davies, J.T., Proc. 2nd Internat. Cong. Surface Activity, 1957, London 1: 426.

- P. Ertl, B. Rohde, P. Selzer, J.Med.Chem., 2000, 43: 3714 was used for the PSA calculations used in this paper.  Other PSA references include P. Stenberg, K. Luthman, H. Ellens, C.P. Lee, P.L. Smith, A. Lago, J.D. Ellito and P. Arturrson, Pharm. Res. 1999, 16:1520 and D.E. Clark, J. Pharm. Sci. 1999, 88: 807.  Intestinal adsportion is covered in K. Palm, et.al., Pharm. Res. 1997, 14: 568.

- Griffin, W.C. J. Soc. Cosmetic Chemists 1949, 1: 311.

- Griffin, W.C. J. Soc. Cosmetic Chemists 1954, 5: 249.

- Hansch, C. and A. Leo, "Substituent Constants for Correlation Analysis in Chemistry and Biology", Wiley and Sons, New York, 1979

- Hansch, C., J.E. Quinlan and G.L. Lawrence, J. Organic Chem. 1968, 33: 347

- Hansen, C.M., J. Paint Technol. 1967, 39: 104-117.

- Kamlet, M.J., J-L.M. Abboud and R.W. Taft, Progress in Physical Organic Chem. 13: 485

- Kamlet M.J., R.M. Doherty, J-L.M. Abboud, M.H. Abraham and R.W. Taft, J. Pharmaceutical Sciences, 1986, 75: 338.

- G. Klopman, S. Wang and D.M. Balthasar, J. Chem. Inf. Comput. Sci, 1992,. 32:474.

- Leahy, D.E., J. Pharmaceutical Sci., 1986, 75: 629.

- Leo, A., C. Hansch and D. Elkins, Chemical Reviews, 1971, 71: 525.

- Lin, I.J. and L. Marszall Progress in Colloid and Polymer Science, 1978, 63: 99.

- Little, R.C. J. Colloid Interface Sci., 1978, 65: 587.

- Lyman, W.J., W.F. Rieehl and D.H. Rosenblatt, "Handbook of Chemical Property Estimation Methods", Mc-Graw-Hill, New York, 1982.

- McGowan, J.C., Tenside Surf. Det., 1990, 27: 229.

- Van Krevelen, D.W., "Properties of Polymers", 3rd edition, Elsevier Science B.V., Amsterdam, 1990.

- V.N. Viswanadhan, A.K. Ghose, G.R. Revankar and R.K. Robins, J. Chem. Inf. Comput. Sci., 1988, 29: 163.

- Yalkowsky, S.H. and R. Pinal, Chemosphere, 1993, 26: 1239.

Figure 1a. Plot of Percent Hydrophilic Surface Area (calculated by the new method) and HLB values from the literature.  The outliers above the line are mainly block copolymers (although most of these are well correlated) and anionic materials.  The outliers below the line contain anionic and cationic materials.  Additional terms to the regression model to account for sodium, potassium and ammonium ions are needed for high correlation.  About 80% of the points are within 1 HLB unit of the regression line.

Figure 1b.  The correlation of hydrophilic surface area and polar surface area is quite pronounced for a group of over 859 gasses, solvents, polymers and surfactants with PSA <2000.  The relationship begins to break down somewhat for highly polar polymers with PSA > 2000.

Figure 2.  octoxynol-4 colored by atom type (top, C = green, O = red, H = white) and by hydrophilic surface value (bottom, blue = lipophilic, red = hydrophilic, white = neutral, darker red is more hydrophilic than lighter red).  The drawings are from the program Molecular Modeling Pro.

Figure 3.  Examples of insecticides with hydrophilic cores or mid-regions and hydrophobic exteriors.  From the top left the molecules are chlorpyrifos, cypermethrin, diazinon, esfenvalerate, flufenoxuron, rotenone and tebufenozide.  Each is representative of a class of molecules with similar hydrophilic profiles.  There are no corresponding insecticides with hydrophobic cores and hydrophilic exteriors that the author is aware of.  All molecules are placed so that their largest dimension is along the x axis (left to right) and next largest dimension along the y axis (top to bottom).  All molecules have a maximum dimension between 1.36 to 1.96 nm and a minimum dimension of 0.72 to 0.87 nm.  The relative sizes of the hydrophilic and hydrophic regions varies considerably (rotenone HSA=4.27, %HSA =17.1 to tebufenozide HSA=8.72, %HSA=41.5).  Thus the placement of the hydrophilic and lipophilic regions are probably more important than the HSA and percent HSA values.

Figure 4.  Insecticides with the molecule divided into a hydrophilic region (left, red) and a lipophilic region (right, blue) by the adjusted hydrophilic atom contributions.  The molecules are from the top left, carbaryl, carbofuran, aldicarb and imidacloprid.  These molecules are all smaller than the molecules in figure 3 with the maximum dimension being 1.13-1.28 nm and the minimum dimension being 0.53-0.65 nm.  Molecules are placed with the maximum dimension on the x axis.

Figure 5. The twenty essential amino acids colored by the adjusted hydrophilicity of each atom.  Top row: Ala, Arg,  Asn, Asp, Cys, second row: Gln, Glu, Gly, His, Ile; Third row: Leu, Lys, Met, Phe, Pro, bottom row: Ser, Thr, Trp, Tyr, Val.  Dark blue is most lipophilic, dark red most hydrophilic.  All molecules are oriented so that the maximum dimension of the molecule is left to right (along the x axis) and the maximum width 90 degrees from the maximum dimension is from top to bottom (along the y axis). The minimum dimension is <0.48-0.6 nm and the maximum dimension within the range 0.70 - 1.19 nm, similar to the dimensions of many of the carbamate insecticides.  Drawing created by the

computer program Molecular Modeling Pro.

| Table 2-1. STIMULANTS, ANTI-DEPRESSANTS, ANTI-PSYCHOTICS and NOOTROPIC Drugs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Percent hydrophilic Surface area | Hydrophilic surface area (cm2/mol x 109) |
| aceclidine | 45.00 | 5.80 |
| aceglutamide | 85.51 | 12.41 |
| acetophenazine | 49.40 | 13.60 |
| acetylcarnitine | 87.90 | 15.27 |
| acetylcholine | 84.82 | 11.86 |
| acetylcholine bromide | 42.41 | 5.93 |
| acetylcholine chloride | 42.41 | 5.93 |
| acetylpheneturide | 44.22 | 7.91 |
| adenosine | 100.00 | 22.67 |
| adinazolam | 32.18 | 6.90 |
| adrafinil | 39.21 | 7.21 |
| albutoin | 49.96 | 8.02 |
| aldosterone | 36.67 | 9.41 |
| alizapride | 56.67 | 12.17 |
| aloxidone | 60.66 | 6.85 |
| amantadine | 26.16 | 3.00 |
| amineptine | 33.20 | 8.06 |
| g-aminobutyric acid (GABA) | 100.00 | 9.05 |
| aminoglutethimide | 76.35 | 12.53 |
| amisulpride | 54.71 | 14.44 |
| amitriptyline | 0.75 | 0.15 |
| amitriptylinoxide | 40.23 | 8.49 |
| amoxapine | 51.27 | 9.39 |
| amphetamine | 33.78 | 3.70 |
| amphetaminil | 26.53 | 4.84 |
| angiotensin II | 71.56 | 65.93 |
| aniracetam | 45.55 | 6.62 |
| arginine vasopressin | 85.69 | 63.74 |
| arginine vasopressin | 85.69 | 63.74 |
| ascorbic acid [vitamin C] | 100.00 | 11.81 |
| aspartate | 100.00 | 9.78 |
| atrolactamide | 42.31 | 5.11 |
| atropine | 31.68 | 6.67 |
| baclofen | 58.62 | 8.52 |
| beclamide | 40.91 | 5.55 |
| bemegride | 40.16 | 4.96 |
| benactyzine | 29.77 | 7.26 |
| bencyclane | 5.55 | 1.32 |
| benmoxine | 37.09 | 6.07 |
| benperidol | 38.55 | 9.46 |
| benserazide | 97.56 | 17.57 |
| benzphetamine | 0.83 | 0.15 |
| bespiridine | 40.32 | 6.66 |
| bethanechol chloride | 92.90 | 7.52 |
| bethanidine | 65.02 | 9.04 |
| bezitramide | 33.63 | 11.19 |
| bifemelane hydrochloride | 55.84 | 7.56 |
| binedaline | 12.98 | 2.72 |
| biperiden | 13.28 | 3.05 |
| bromocriptine | 48.02 | 18.95 |
| bromperidol | 29.17 | 7.31 |
| brucine | 23.35 | 5.71 |
| budipine | 0.69 | 0.15 |
| bupropion | 18.51 | 3.31 |
| buramate | 65.76 | 9.34 |
| buramate | 65.76 | 9.34 |
| butacetin | 28.09 | 4.53 |
| butaperazine | 36.22 | 10.26 |
| butriptyline | 0.69 | 0.15 |
| n-butylnitrite | 31.52 | 2.86 |
| caffeine | 54.50 | 6.92 |
| calcium bromide | 100.00 | 3.85 |
| carbachol | 100.00 | 7.90 |
| carbamazepine | 42.12 | 5.96 |
| carbidopa | 91.01 | 7.75 |
| caroxazone | 69.07 | 9.01 |
| carphenazine | 46.72 | 13.62 |
| carpipramine | 33.64 | 10.72 |
| carpronium choride | 93.13 | 7.76 |
| 2C-B | 30.35 | 4.76 |
| 2C-D | 30.72 | 4.76 |
| chlorphentermine | 15.64 | 2.12 |
| chlorproethazine | 5.98 | 1.37 |
| chlorprothixene | 0.77 | 0.15 |
| choline | 100.00 | 11.01 |
| choline alfoscerate | 100.00 | 20.61 |
| ciclonicate | 34.96 | 6.57 |
| cinnarizine | 26.07 | 6.65 |
| cinromide | 37.34 | 5.48 |
| citalopram | 16.68 | 3.74 |
| citicoline | 100.00 | 32.67 |
| clocapramine | 32.51 | 10.72 |
| clofenciclan | 15.51 | 3.68 |
| clomacran | 13.01 | 2.61 |
| clomethiazole | 4.63 | 0.48 |
| clomipramine | 0.44 | 0.10 |
| clonazepam | 55.38 | 9.63 |
| clonidine | 41.57 | 5.10 |
| clopenthixol | 42.54 | 10.89 |
| clortermine | 15.64 | 2.12 |
| clospirazine | 47.56 | 12.68 |
| clothiapine | 18.71 | 3.78 |
| clozapine | 43.28 | 8.60 |
| cocaine | 35.22 | 7.36 |
| corticosterone | 29.66 | 7.55 |
| cortisol | 34.22 | 9.02 |
| cotinine | 34.51 | 4.67 |
| cyamemazine | 18.37 | 4.02 |
| cyclandelate | 22.49 | 4.77 |
| cycloserine | 100.00 | 7.26 |
| cyproheptadine | 0.65 | 0.12 |
| deanol | 40.04 | 3.56 |
| debrisoquin | 58.62 | 7.23 |
| decimemide | 25.20 | 6.83 |
| dehydroepiandrosterone (DHEA) | 11.84 | 2.58 |
| demanyl phosphate | 59.36 | 7.79 |
| demecarium bromide | 87.29 | 11.34 |
| demexiptiline | 29.61 | 5.60 |
| desipramine | 32.58 | 6.33 |
| dexetimide | 29.97 | 7.42 |
| dexpanthenol | 75.78 | 13.17 |
| diethadione | 48.00 | 6.35 |
| diethazine | 18.42 | 3.72 |
| diethylpropion | 9.93 | 1.65 |
| dihydroergocornine methanesulfonate | 75.20 | 12.44 |
| dihydroergocristine methanesulfonate | 74.60 | 12.62 |
| dihydro-a-ergocryptine methanesulfonate | 74.87 | 12.65 |
| dihydro-b-ergocryptine methanesulfonate | 74.33 | 12.44 |
| diisopropylamine dichloroacetate | 54.46 | 4.24 |
| diisopropyl paraoxon | 32.93 | 6.93 |
| dimetacrine | 0.43 | 0.10 |
| dimethadione | 53.77 | 4.90 |
| dimethazan | 50.25 | 8.97 |
| 2,5-dimethoxy-4-bromoamphetamine | 21.74 | 3.70 |
| 2,5-dimethoxy-4-methylamphetamine | 21.98 | 3.70 |
| dimethylphenylpiperazinium (DMPP) | 68.42 | 10.53 |
| dioxadrol | 39.97 | 8.32 |
| dioxadrol | 38.43 | 8.03 |
| disulfiram | 34.88 | 7.58 |
| dixyrazine | 50.82 | 15.16 |
| dizocilpine | 25.34 | 3.53 |
| dopamine | 80.10 | 9.03 |
| dothiepin | 0.48 | 0.10 |
| doxepin | 3.44 | 0.69 |
| droperidol | 43.04 | 10.36 |
| droxidopa | 90.33 | 13.17 |
| duloxetine | 28.12 | 5.44 |
| dynorphin | 80.06 | 126.97 |
| E-2020 | 53.98 | 2.32 |
| eburnamonine | 26.73 | 5.07 |
| echothiophate iodide | 85.74 | 9.29 |
| edrophonium chloride | 91.74 | 7.03 |
| B-endorphin | 75.69 | 193.45 |
| epinephrine | 78.58 | 10.53 |
| eptastigmine | 25.96 | 7.17 |
| eseridine | 32.34 | 6.64 |
| 17-B-estradiol | 15.01 | 2.88 |
| estriol | 26.23 | 5.24 |
| estrone | 13.72 | 2.56 |
| eterobarb | 43.38 | 9.76 |
| ethadione | 46.70 | 5.53 |
| ethamivan | 30.66 | 5.18 |
| ethanol | 28.83 | 1.43 |
| ethopropazine | 6.30 | 1.37 |
| ethosuximide | 49.60 | 5.46 |
| ethotoin | 59.71 | 8.09 |
| N-ethylamphetamine | 22.26 | 3.05 |
| ethylbenzhydramine | 17.52 | 3.79 |
| etifelmin | 20.48 | 3.53 |
| etoperidone | 36.59 | 9.65 |
| etryptamine | 54.54 | 7.40 |
| exifone | 94.75 | 15.83 |
| fasudil | 80.72 | 14.97 |
| febarbamate | 49.24 | 14.22 |
| felbamate | 66.55 | 11.16 |
| femoxetine | 3.38 | 0.76 |
| fencamfamine | 19.14 | 3.05 |
| fencamine | 39.67 | 10.87 |
| fenethylline | 48.43 | 11.38 |
| fenoxedil | 20.08 | 7.57 |
| fenozolone | 49.14 | 6.73 |
| fenpentadiol | 16.85 | 2.86 |
| fipexide | 55.64 | 13.51 |
| fluacizine | 24.40 | 5.52 |
| fluanisone | 26.11 | 6.43 |
| flunarizine | 25.18 | 6.65 |
| fluoresone | 26.67 | 3.22 |
| fluoxetine | 26.75 | 5.44 |
| flupentixol | 39.91 | 10.90 |
| fluphenazine | 43.61 | 12.12 |
| flurothyl | 15.92 | 1.71 |
| fluspirilene | 26.83 | 8.30 |
| fluvoxamine | 28.63 | 6.33 |
| furtrethonium | 98.21 | 7.61 |
| gabapentin | 53.65 | 7.73 |
| glutamate | 100.00 | 11.10 |
| guanabenz | 47.92 | 6.60 |
| guanacline | 56.11 | 8.60 |
| guanadrel | 58.62 | 9.46 |
| guanazodine | 65.99 | 10.37 |
| guanethidine | 50.23 | 8.60 |
| guanfacine | 49.91 | 7.44 |
| haloperidol | 29.66 | 7.31 |
| haloperidol | 29.48 | 7.31 |
| harmaline | 39.53 | 5.41 |
| hematoporphyrin | 62.02 | 24.49 |
| hexacyclonate sodium | 54.98 | 9.28 |
| histamine | 100.00 | 8.47 |
| homocamfin | 8.78 | 1.15 |
| g-hydroxybutyrate | 89.49 | 7.85 |
| 5-hydroxytryptophan | 95.36 | 13.51 |
| hypericin | 70.30 | 17.62 |
| ibogaine | 22.56 | 4.72 |
| ibudilast | 21.69 | 3.63 |
| idebenone | 26.97 | 7.25 |
| ifenprodil | 14.38 | 3.38 |
| imiclopazine | 51.32 | 16.19 |
| picrotoxin | 0.47 | 0.10 |
| imipramine n-oxide | 46.30 | 10.50 |
| indalpine | 49.31 | 8.08 |
| indeloxazine hydrochloride | 52.56 | 8.41 |
| indeloxazine hydrochloride | 77.27 | 5.60 |
| iprindole | 6.46 | 1.40 |
| iproclozide | 39.53 | 6.67 |
| iproniazid | 64.88 | 8.59 |
| isocarboxazid | 45.82 | 7.02 |
| isoflurophate | 31.90 | 4.58 |
| isoproterenol | 62.49 | 10.13 |
| ketamine | 33.43 | 5.49 |
| lamotrigine | 48.52 | 6.76 |
| lazabemide | 74.78 | 9.84 |
| lecithin | 31.78 | 22.41 |
| lecithin | 31.78 | 22.41 |
| leucine-enkephalin | 63.58 | 25.31 |
| levodopa | 84.28 | 11.60 |
| levophacetoperane | 42.91 | 7.30 |
| lobeline | 15.46 | 3.74 |
| lofepramine | 13.98 | 3.82 |
| lomerizine | 24.95 | 7.82 |
| loxapine | 24.94 | 4.91 |
| lysergide | 34.04 | 7.55 |
| maprotiline | 23.16 | 4.56 |
| mazindol | 47.95 | 7.74 |
| MDA | 58.75 | 7.39 |
| MDE | 37.29 | 5.72 |
| MDMA | 46.35 | 6.49 |
| mecamylamine | 21.61 | 3.16 |
| meclofenoxate | 35.71 | 6.52 |
| medifoxamine | 12.00 | 2.22 |
| mefexamide | 36.05 | 8.00 |
| melatonin | 53.56 | 8.69 |
| melitracen | 0.70 | 0.15 |
| melperone | 11.02 | 2.16 |
| memantine | 12.08 | 1.76 |
| mepazine | 4.22 | 0.85 |
| mephenytoin | 57.61 | 8.67 |
| mephobarbital | 50.17 | 8.36 |
| mescaline | 29.00 | 4.76 |
| mesoridazine | 13.42 | 3.31 |
| metapramine | 33.98 | 5.70 |
| methacholine chloride | 86.16 | 6.65 |
| methamphetamine | 30.91 | 3.82 |
| metharbital | 53.94 | 8.06 |
| methetoin | 38.42 | 5.78 |
| methionine-enkephalin | 64.53 | 25.74 |
| methoxypromazine | 7.80 | 1.67 |
| methsuximide | 32.99 | 4.66 |
| a-methyldopa | 80.81 | 12.28 |
| 5-methyl-5-(3-phenanthryl)hydantoin | 44.83 | 7.57 |
| methylphenidate | 40.85 | 7.09 |
| 3-methyl-5-phenylhydantoin | 72.54 | 8.86 |
| metofenazate | 37.80 | 14.67 |
| metralindole | 42.36 | 6.86 |
| mianserin | 9.73 | 1.70 |
| milnacipran | 31.03 | 5.91 |
| minaprine | 64.42 | 13.55 |
| mirtazepine | 21.70 | 3.76 |
| moclobemide | 75.11 | 13.59 |
| modafinil | 38.57 | 6.76 |
| mofegiline | 38.33 | 5.45 |
| molindone | 54.96 | 11.08 |
| moperone | 29.38 | 7.31 |
| muscarine | 83.59 | 13.07 |
| nafronyl | 24.20 | 6.91 |
| narcobarbital | 43.29 | 8.07 |
| nebracetam | 65.16 | 9.64 |
| nefazodone | 37.19 | 11.83 |
| nefiracetam | 51.80 | 9.12 |
| nefopam | 23.24 | 4.11 |
| neostigmine | 87.27 | 8.25 |
| neuropeptide Y | 78.37 | 234.30 |
| neurotensin | 77.50 | 96.50 |
| nialamide | 71.83 | 14.59 |
| nicametate | 55.37 | 9.59 |
| nicergoline | 34.46 | 9.67 |
| nicotine | 20.58 | 2.48 |
| nicotinic acid | 94.68 | 7.69 |
| nikethamide | 43.34 | 5.92 |
| nimetazepam | 39.73 | 7.03 |
| nimodipine | 43.38 | 12.70 |
| nitrazepam | 60.76 | 9.85 |
| nizofenone | 35.94 | 9.37 |
| nomifensine | 40.05 | 6.64 |
| norepinephrine | 87.88 | 10.57 |
| nortriptyline | 27.06 | 5.01 |
| noxiptilin | 7.53 | 1.53 |
| octamoxin | 38.41 | 5.50 |
| opipramol | 46.51 | 11.78 |
| opipramol | 47.84 | 12.12 |
| oxaflozane | 30.92 | 5.76 |
| oxaflumazine | 49.03 | 15.96 |
| oxapropanium iodide | 100.00 | 8.23 |
| oxcarbazepine | 52.75 | 8.01 |
| oxiracetam | 100.00 | 11.42 |
| oxitriptan | 96.95 | 13.93 |
| oxotremorine | 60.94 | 9.91 |
| oxypertine | 36.16 | 9.57 |
| oxytocin | 75.43 | 53.69 |
| papaverine | 8.26 | 1.88 |
| paramethadione | 41.54 | 4.92 |
| pargyline | 1.18 | 0.15 |
| paroxetine | 44.20 | 9.20 |
| pemoline | 55.66 | 6.10 |
| penfluridol | 16.44 | 5.27 |
| pentifylline | 43.11 | 8.35 |
| pentylenetetrazole | 48.82 | 4.69 |
| perazine | 34.09 | 7.77 |
| pergolide | 17.73 | 3.80 |
| pericyazine | 37.95 | 9.04 |
| perimethazine | 19.83 | 5.15 |
| perphenazine | 46.43 | 12.12 |
| phenacemide | 60.48 | 7.44 |
| phencyclidine | 0.82 | 0.16 |
| phendimetrazine | 20.35 | 2.94 |
| phenelzine | 54.32 | 5.78 |
| phenetharbital | 42.96 | 7.71 |
| phenethylamine | 49.57 | 4.76 |
| pheneturide | 42.61 | 6.38 |
| phenmetrazine | 46.61 | 6.08 |
| phenobarbital | 58.17 | 8.87 |
| phensuximide | 41.27 | 5.17 |
| phentermine | 17.10 | 2.12 |
| phenylmethylbarbituric acid | 49.85 | 7.01 |
| phenytoin | 56.34 | 8.66 |
| phethenylate sodium | 47.66 | 8.37 |
| physostigmine | 38.38 | 7.54 |
| piberaline | 55.50 | 10.55 |
| picrotin | 57.47 | 11.02 |
| picrotoxin |  |  |
| picrotoxinin | 42.80 | 8.33 |
| pilocarpine | 41.30 | 6.31 |
| pimozide | 25.53 | 7.47 |
| pipamperone | 44.78 | 12.21 |
| piperacetazine | 19.20 | 5.36 |
| pipotiazine | 30.74 | 9.83 |
| pipradrol | 33.71 | 6.35 |
| piracetam | 91.31 | 9.68 |
| piroheptine | 0.71 | 0.15 |
| pizotyline | 0.78 | 0.15 |
| potassium bromide | 100.00 | 4.40 |
| pramipexole | 65.54 | 10.04 |
| pramiracetam | 52.51 | 11.79 |
| pridinol | 16.08 | 3.47 |
| primidone | 58.09 | 8.84 |
| prochlorperazine | 32.44 | 7.77 |
| prodipine | 0.73 | 0.15 |
| progabide | 44.09 | 9.36 |
| progesterone | 11.31 | 2.69 |
| prolactin | 11.31 | 2.69 |
| prolintane | 6.65 | 1.17 |
| prolintane | 6.02 | 1.06 |
| promazine | 7.14 | 1.36 |
| propentofylline | 44.81 | 9.99 |
| propizepine | 53.33 | 10.99 |
| propranolol | 35.62 | 6.77 |
| prothipendyl | 27.52 | 5.22 |
| protriptyline | 23.37 | 4.50 |
| psilocybin | 56.48 | 10.61 |
| pyridostigmine bromide | 83.93 | 6.13 |
| pyridoxine hydrochloride | 88.86 | 6.05 |
| pyrisuccideanol | 68.56 | 17.59 |
| pyritinol | 69.84 | 16.94 |
| pyrovalerone | 18.72 | 3.57 |
| quinupramine | 14.09 | 2.82 |
| ritanserin | 18.90 | 5.49 |
| rolicyprine | 68.05 | 10.97 |
| rolipram | 51.41 | 9.86 |
| ropinirole | 27.41 | 5.52 |
| roxindole | 30.84 | 7.20 |
| rubidium chloride | 100.00 | 2.96 |
| sabeluzole | 30.74 | 8.28 |
| scopolamine | 53.32 | 10.91 |
| selegiline | 0.97 | 0.15 |
| serotonin | 96.36 | 11.59 |
| sertraline | 20.89 | 3.97 |
| sodium bromide | 100.00 | 3.47 |
| spiperone | 46.43 | 12.21 |
| spirilene | 30.80 | 8.27 |
| strontium bromide | 100.00 | 4.06 |
| Substance P | 78.02 | 76.95 |
| suclofenide | 58.84 | 12.26 |
| sulforidazine | 24.35 | 6.32 |
| sulpiride | 61.55 | 14.65 |
| sulpiride | 61.55 | 14.65 |
| sulthiame | 77.47 | 13.86 |
| tacrine | 40.88 | 5.27 |
| talipexole | 49.00 | 7.30 |
| tandospirone | 67.08 | 17.40 |
| terguride | 54.58 | 12.96 |
| testosterone | 13.91 | 3.01 |
| tetrabenazine | 14.47 | 3.41 |
| tetrahydrocannabinols (THC) | 5.84 | 1.42 |
| tetrantoin | 55.97 | 7.72 |
| thiamine hydrochloride | 92.09 | 6.22 |
| thiazesim | 13.96 | 3.12 |
| thiopropazate | 44.78 | 13.04 |
| thioproperazine | 46.21 | 13.74 |
| thioridazine | 5.62 | 1.37 |
| thiothixene | 39.49 | 11.55 |
| tiagabine | 29.77 | 7.59 |
| tianeptine | 44.89 | 12.34 |
| timiperone | 40.04 | 10.07 |
| tinofedrine | 28.36 | 6.50 |
| tinofedrine | 28.36 | 6.50 |
| tofenacin | 35.89 | 6.80 |
| toloxatone | 55.70 | 7.81 |
| topiramate | 62.26 | 13.87 |
| tranylcypromine | 48.37 | 4.55 |
| trazodone | 47.37 | 11.18 |
| trifluoperazine | 30.30 | 7.77 |
| trifluperidol | 28.03 | 7.31 |
| triflupromazine | 6.22 | 1.36 |
| trihexyphenidyl hydrochloride | 56.45 | 2.80 |
| trimethadione | 46.81 | 4.92 |
| trimethapan camsylate | 47.57 | 9.38 |
| L-tryptophan | 75.90 | 10.24 |
| valproic acid | 28.25 | 3.76 |
| valpromide | 29.08 | 3.91 |
| velnacrine | 59.43 | 8.14 |
| venlafaxine | 7.32 | 1.62 |
| vigabatrin | 91.82 | 10.34 |
| viloxazine | 49.40 | 8.66 |
| vincamine | 32.03 | 7.50 |
| vinconate | 21.42 | 4.17 |
| vinpocetine | 28.85 | 6.78 |
| viquidil | 37.69 | 8.65 |
| xanomeline | 24.12 | 5.14 |
| xanomeline | 24.12 | 5.14 |
| zimeldine | 10.59 | 1.99 |
| zometapine | 40.12 | 6.97 |
| zonisamide | 52.94 | 6.60 |
| zotepine | 3.73 | 0.76 |

| Table 2-2. PESTICIDES |  |  |
| --- | --- | --- |
| Name | Percent hydrophilic surface area | Hydrophilic surface area (cm2/mol x 109) |
| abamectin B1a | 40.55 | 26.00 |
| abamectin B1b | 41.41 | 26.00 |
| acephate | 72.00 | 9.04 |
| acetimiprid | 42.96 | 6.61 |
| acetochlor | 21.04 | 4.17 |
| acifluorfen sodium | 67.97 | 5.40 |
| acinathrin | 25.68 | 8.51 |
| aclonifen | 44.36 | 6.82 |
| ACN, quinoclamine | 71.94 | 8.33 |
| acrisorcin | 38.02 | 4.85 |
| acrolein | 43.74 | 2.27 |
| aculeacin | 59.84 | 46.51 |
| akton | 25.17 | 5.05 |
| alachlor | 21.04 | 4.17 |
| alantolactone | 15.67 | 2.69 |
| alanycarb | 42.22 | 11.96 |
| ametryn | 46.95 | 7.89 |
| amidithion | 64.74 | 12.14 |
| amidosulfuron | 73.57 | 16.11 |
| amidothionate | 36.09 | 7.03 |
| o-aminoacetophenone | 72.22 | 7.13 |
| aminocarb | 50.08 | 7.29 |
| aminofuracarb | 33.55 | 9.19 |
| amoscanate | 76.63 | 12.16 |
| amoscanate | 76.20 | 12.58 |
| ampropylfos | 79.51 | 8.18 |
| annonin I | 20.53 | 10.67 |
| anthiolimine | 100.00 | 30.94 |
| anthraquinone | 32.07 | 3.79 |
| aldicarb | 40.84 | 6.17 |
| aldoxycarb | 58.26 | 9.51 |
| aldrin | 0.00 | 0.00 |
| allethrin | 16.36 | 3.97 |
| allosamidin | 80.87 | 36.41 |
| alloxydim-sodium | 80.87 | 36.41 |
| allyxycarb | 26.54 | 5.73 |
| altemicidin | 83.21 | 17.78 |
| 5-amino-3-cyano-1-(2,6-dichloro-trifluoromethylphenyl)-4-trifluoromethylsulfinylpyrazole | 38.87 | 8.29 |
| amiphos | 31.35 | 7.22 |
| amiphos (DAEP) | 60.84 | 10.10 |
| amitraz | 16.08 | 3.54 |
| amitrole | 100.00 | 5.49 |
| ammonium sulfamate | 100.00 | 7.24 |
| ammonium thiocyanate | 100.00 | 5.87 |
| amorolfine | 6.74 | 1.80 |
| amphotalide | 61.06 | 12.97 |
| amphotericin B | 54.06 | 37.71 |
| ancymidol | 37.12 | 6.28 |
| andoprim | 37.25 | 5.90 |
| angulatin A | 45.52 | 21.96 |
| anilazine | 37.59 | 5.44 |
| anilofos | 33.13 | 7.84 |
| anisylacetone | 8.10 | 1.11 |
| antimycin A | 54.60 | 20.54 |
| aramite | 28.72 | 6.78 |
| arecoline | 33.53 | 4.19 |
| aristolochic acid | 54.21 | 10.22 |
| arthrichitin | 57.44 | 27.22 |
| aspidin | 35.88 | 12.09 |
| aspidinol | 28.95 | 4.84 |
| aspon | 38.18 | 9.97 |
| asulam | 90.28 | 12.83 |
| atrazine | 55.07 | 8.31 |
| azaconazole | 56.19 | 9.39 |
| 2-aza-2,3-dihydrosqualene | 0.25 | 0.10 |
| azadirachtin | 62.13 | 29.19 |
| azamethiphos | 61.81 | 11.27 |
| azaserine | 100.00 | 11.81 |
| 15-azasterol | 6.17 | 2.09 |
| azinphos-ethyl | 61.16 | 13.38 |
| azinphos-methyl | 63.72 | 12.24 |
| aziprotryn | 55.08 | 8.35 |
| azithiram | 70.26 | 13.07 |
| azobenzene | 16.80 | 2.03 |
| azocyclotin | 14.78 | 3.80 |
| azoxystrobin | 43.44 | 11.05 |
| bagougeramine A | 100.00 | 33.23 |
| barban | 46.17 | 7.40 |
| barthrin | 26.99 | 5.80 |
| becanthone | 33.14 | 8.77 |
| benalaxyl | 19.11 | 4.51 |
| bensulfuron-methyl | 63.05 | 15.08 |
| bensulide | 51.40 | 13.51 |
| bensultap | 55.76 | 14.82 |
| bentazone | 48.80 | 6.96 |
| bentranil | 43.43 | 5.75 |
| benzadox | 73.36 | 9.52 |
| benzofenap | 25.48 | 6.79 |
| benzotemap | 33.76 | 7.45 |
| benzoximate | 18.96 | 4.45 |
| benzoyl chloride | 28.48 | 5.08 |
| benzoylprop-ethyl | 22.20 | 5.12 |
| benzquinox | 51.56 | 7.98 |
| benzthiazuron | 67.94 | 8.51 |
| 6-benzylaminopurine | 62.71 | 8.47 |
| bifonazole | 21.31 | 4.25 |
| bifonazole | 21.82 | 4.36 |
| binapacryl | 31.67 | 7.15 |
| benazolin | 61.49 | 8.20 |
| bendiocarb | 46.42 | 6.92 |
| benefin | 22.85 | 4.92 |
| benfuracarb | 34.44 | 10.19 |
| benfuresate | 33.66 | 5.68 |
| benodanil | 30.86 | 4.67 |
| benomyl | 58.81 | 11.69 |
| benoxacor | 20.36 | 3.19 |
| bollex | 8.53 | 2.25 |
| bomyl | 55.52 | 11.02 |
| brasassian | 34.29 | 4.94 |
| brodifacoum | 17.27 | 4.90 |
| bromacil | 30.86 | 4.67 |
| benzyl benzoate | 27.05 | 3.86 |
| bephenium chloride | 79.14 | 6.90 |
| bephenium napthoate | 55.00 | 8.68 |
| BHC (hexachlorocyclohexane) | 0.00 | 0.00 |
| bialaphos | 80.89 | 14.97 |
| bifenox | 28.24 | 5.42 |
| bifenthrin | 13.18 | 3.56 |
| bioallethrin | 16.36 | 3.97 |
| biphenamine | 35.47 | 8.13 |
| bispyribac-sodium | 39.80 | 9.63 |
| bitertanol | 30.47 | 7.11 |
| bitoscanate | 64.77 | 7.48 |
| blasticidin-S | 100.00 | 29.54 |
| bromadiolone | 22.57 | 6.58 |
| bromethalin | 20.54 | 4.95 |
| bromobutide | 11.50 | 2.44 |
| bromophenoxim | 27.67 | 5.72 |
| bromophos | 22.99 | 4.06 |
| bromophos ethyl | 19.98 | 4.06 |
| bromopropylate | 19.24 | 4.27 |
| bromosalicylchloranilide | 43.70 | 7.22 |
| bromoxynil | 27.15 | 3.08 |
| brompyrazon | 44.52 | 5.71 |
| bromuconazole | 33.67 | 6.47 |
| buclosamide | 42.42 | 6.76 |
| bufencarb | 37.34 | 6.58 |
| bulbosan | 48.48 | 7.41 |
| bupirimate | 44.36 | 10.02 |
| buprofezin | 27.20 | 6.06 |
| butacarb | 26.05 | 5.77 |
| butachlor | 17.51 | 4.17 |
| butam | 10.65 | 2.08 |
| butamifos | 42.53 | 9.71 |
| butenachlor | 20.01 | 4.67 |
| butenafine | 4.05 | 0.96 |
| buthidazole | 58.36 | 10.48 |
| buthiobate | 27.94 | 7.65 |
| butocarboxim | 41.12 | 6.08 |
| butoconazole | 29.48 | 6.97 |
| butonate | 28.83 | 5.97 |
| butoxycarboxim | 54.39 | 8.75 |
| 2-(2-butoxyethoxy)ethylthiocyanate | 53.54 | 9.38 |
| calciferol | 4.19 | 1.43 |
| calcium cyanide | 100.00 | 5.66 |
| calcium propionate | 63.47 | 9.12 |
| candicidin D | 53.20 | 44.44 |
| carbofuran | 42.29 | 6.59 |
| carbon tetrachloride | 0.00 | 0.00 |
| carbophenothion | 34.43 | 7.28 |
| carbosulfan | 21.62 | 6.22 |
| carboxin | 53.51 | 8.46 |
| 3-(2-carboxyphenyl)-5-phenylpyrazole | 55.04 | 8.93 |
| carpropamid | 16.07 | 3.44 |
| chaetiacandin | 55.50 | 33.55 |
| chinomethionat | 56.46 | 7.10 |
| butralin | 32.18 | 7.05 |
| butrizol | 43.95 | 4.48 |
| buturon | 54.04 | 8.17 |
| sec-butylamine | 29.62 | 2.34 |
| butylate | 16.25 | 3.07 |
| cacodylic acid | 42.09 | 3.14 |
| cadusafos | 26.54 | 5.53 |
| cafenstrole | 37.68 | 9.09 |
| candidin | 54.40 | 38.22 |
| captafol | 31.11 | 5.90 |
| captan | 35.85 | 5.89 |
| carbamorph | 72.91 | 11.72 |
| carbanolate | 39.09 | 5.68 |
| carbaryl | 50.99 | 6.75 |
| carbendazim | 69.33 | 8.19 |
| carbetamide | 55.41 | 9.53 |
| carfentrazone-ethyl | 37.69 | 8.92 |
| cartap | 71.97 | 12.41 |
| carvacrol | 6.44 | 0.79 |
| casbene | 21.01 | 3.72 |
| CDAA | 56.16 | 7.62 |
| CDEC | 26.30 | 4.28 |
| CECA | 100.00 | 10.64 |
| 3-CF3-sulfonamido-p-acetotoluide | 69.11 | 11.99 |
| chinosol | 69.83 | 4.52 |
| chlobenthiazone | 33.76 | 3.74 |
| chlomethoxynil | 13.94 | 2.46 |
| chloralose | 67.85 | 11.81 |
| chloramben | 82.59 | 9.54 |
| chloraniformethane | 37.72 | 6.85 |
| chloranil | 25.57 | 2.94 |
| chlorazifop propynyl | 21.58 | 4.79 |
| chlorbromuron | 32.37 | 5.01 |
| chlorbufam | 34.23 | 5.16 |
| chlordane | 0.00 | 0.00 |
| chlordantoin | 29.34 | 6.47 |
| chlordecone | 5.57 | 1.09 |
| chlordimeform | 12.84 | 1.81 |
| chlorfenac | 33.23 | 4.33 |
| chlorfenethol | 8.73 | 1.44 |
| chlorfenprop methyl | 15.43 | 2.25 |
| chlorfenson | 23.74 | 3.83 |
| chlorfensulphide | 19.28 | 3.48 |
| chlorfensulphide | 19.25 | 3.47 |
| chlorfenvinphos | 26.48 | 5.70 |
| chlorfluazuron | 30.90 | 8.49 |
| chlorflurazole | 54.12 | 6.44 |
| chlorflurenol | 28.20 | 4.56 |
| chloridazon | 61.09 | 7.90 |
| chlorimuron-ethyl | 58.81 | 14.25 |
| chlormephos | 42.43 | 6.42 |
| chlormequat chloride | 100.00 | 13.53 |
| chlormidazole | 20.93 | 3.07 |
| chlornitrofen | 14.80 | 2.46 |
| 5-chloro,3-methyl,4-nitropyrazole | 57.96 | 5.29 |
| chlorothalonil | 33.31 | 4.56 |
| 3-chloro-p-toluidine | 42.05 | 3.98 |
| chloroxuron | 23.64 | 4.42 |
| chloroxynil | 29.11 | 3.09 |
| chlorphenesin | 51.41 | 6.94 |
| chlorphonium | 1.29 | 0.36 |
| chlorphoxim | 35.70 | 7.50 |
| chloroacetic acid | 100.00 | 6.24 |
| chlorobenzilate | 24.18 | 4.88 |
| chloromycorrhizin-a | 32.88 | 6.20 |
| chloroneb | 0.00 | 0.00 |
| chloropicrin | 32.24 | 2.66 |
| chloropon | 73.01 | 7.28 |
| chloropropylate | 19.92 | 4.27 |
| chlorpropham | 34.74 | 5.02 |
| chlorpyrifos | 25.51 | 4.98 |
| chlorpyrifos-methyl | 30.12 | 5.15 |
| chlorquinox | 32.56 | 4.10 |
| chlorsulfuron | 55.70 | 11.39 |
| chlorthal | 52.56 | 7.96 |
| chlorthiamid | 36.73 | 4.22 |
| chlorthiophos | 20.71 | 4.48 |
| chlozolinate | 34.06 | 6.63 |
| ciclopirox | 10.66 | 1.61 |
| cilofungin | 65.11 | 48.00 |
| cinmethylin | 8.16 | 1.72 |
| cinosulfuron | 59.84 | 15.51 |
| clethodim | 25.71 | 5.02 |
| climbazole | 36.97 | 7.31 |
| cloconazole | 25.89 | 5.09 |
| clocythrin | 17.94 | 5.38 |
| cloethocarb | 48.04 | 8.38 |
| clofencet | 41.87 | 7.02 |
| clofentezene | 25.86 | 4.14 |
| clofop isobutyl | 12.65 | 2.96 |
| clomazone | 15.91 | 2.47 |
| clomeprop | 27.30 | 5.48 |
| cloprop | 32.87 | 4.25 |
| clotrimazole | 19.43 | 4.17 |
| cloxyfonac | 62.51 | 8.58 |
| cloxyquin | 35.29 | 3.59 |
| CNP | 14.80 | 2.46 |
| cochlioquinone A | 30.40 | 12.23 |
| compactin | 31.86 | 9.03 |
| conen | 32.02 | 6.85 |
| coparaffinate | 37.12 | 6.61 |
| coroxon | 38.24 | 8.05 |
| coumachlor | 28.87 | 6.04 |
| coumaphos | 35.98 | 7.87 |
| coumatetrayl | 27.22 | 4.92 |
| 3-CPA | 46.92 | 5.44 |
| 4-CPA | 46.92 | 5.44 |
| CPMC | 48.21 | 5.72 |
| crotoxyphos | 33.26 | 7.30 |
| crufomate | 35.81 | 7.32 |
| cyanazine | 62.36 | 10.47 |
| cyanofenphos | 33.49 | 6.49 |
| cyanophos | 43.81 | 6.82 |
| cyclanilide | 61.32 | 9.68 |
| cycloate | 21.47 | 3.73 |
| cycloheximide | 49.06 | 10.20 |
| cycloprothrin | 18.04 | 5.38 |
| cyclosulfamuron | 61.40 | 15.99 |
| cycloxydim | 17.96 | 4.35 |
| cycluron | 33.32 | 5.65 |
| cyfluthrin | 19.85 | 5.38 |
| cyhalofop | 36.18 | 7.13 |
| cyhalofop-butyl | 23.66 | 5.93 |
| cyhalothrin | 19.21 | 5.38 |
| cyhexatin | 4.73 | 1.15 |
| cymoxanil | 71.90 | 10.97 |
| cyometrinil | 51.38 | 6.62 |
| cypendazole | 69.30 | 10.91 |
| cypendazole | 61.84 | 14.06 |
| cypermethrin | 20.25 | 5.38 |
| a-cyperone | 6.33 | 1.12 |
| cyphenothrin | 19.87 | 5.37 |
| cyprazine | 56.08 | 8.32 |
| cyproconazole | 34.18 | 6.68 |
| cyprodinil | 39.02 | 5.98 |
| cyprofuram | 42.65 | 8.46 |
| cypromid | 33.26 | 4.38 |
| cyromazine | 84.11 | 9.57 |
| cythioate | 72.43 | 12.71 |
| 2,4-D | 42.81 | 5.44 |
| dalapon | 76.37 | 6.81 |
| daminozide | 78.45 | 10.07 |
| dazomet | 61.16 | 6.92 |
| 2,4-DB | 31.30 | 4.81 |
| DCPA | 24.41 | 4.36 |
| DCPM | 10.48 | 1.65 |
| DDA | 25.12 | 4.10 |
| DDC-Ni | 48.76 | 8.35 |
| DDCP | 0.00 | 0.00 |
| DDDP | 0.00 | 0.00 |
| DDE | 0.00 | 0.00 |
| DDT | 0.00 | 0.00 |
| decan-1-ol | 12.41 | 1.94 |
| DEET | 12.25 | 1.86 |
| DEF | 25.80 | 6.39 |
| deltamethrin | 19.77 | 5.37 |
| demephion | 34.32 | 5.34 |
| demeton | 37.08 | 6.26 |
| demeton-O-methyl | 45.70 | 7.40 |
| demeton-S-methyl | 34.69 | 5.64 |
| demeton-S-methylsulphon | 57.41 | 10.03 |
| dermostatin A | 40.76 | 23.59 |
| dermostatin B | 39.95 | 23.63 |
| desmedipham | 64.67 | 12.81 |
| desmetryn | 55.99 | 8.66 |
| diafenthiuron | 15.04 | 4.38 |
| dialifos (dialifor) | 49.55 | 11.02 |
| diallate | 18.33 | 3.52 |
| diamate | 40.72 | 5.74 |
| diamidfos | 75.32 | 10.53 |
| diamthazole dihydrochloride | 16.46 | 3.57 |
| diaphene | 32.23 | 5.95 |
| diazinon | 32.48 | 6.98 |
| dibromochloropropane (DBCP) | 0.00 | 0.00 |
| dibutyl phthalate | 35.38 | 7.57 |
| dicamba | 36.07 | 4.59 |
| dicapthon | 38.38 | 6.53 |
| dichlobenil | 23.27 | 2.28 |
| dichlofenthion | 23.83 | 4.49 |
| dichlofluanid | 25.08 | 4.77 |
| dichlone | 35.03 | 4.11 |
| dichlormate | 49.14 | 7.02 |
| o-dichlorobenzene | 0.00 | 0.00 |
| dichlorophen | 17.37 | 2.74 |
| 3,6-dichloropicolinic acid | 46.18 | 4.81 |
| 1,3-dichloropropene | 0.00 | 0.00 |
| dichlorprop | 30.26 | 4.25 |
| dichlorprop-P | 30.39 | 4.28 |
| dichlorvos | 38.52 | 5.08 |
| diclobutrazol | 27.10 | 5.90 |
| diclofop methyl | 13.26 | 2.72 |
| diclomezine | 37.89 | 5.42 |
| dicloran | 49.33 | 5.59 |
| dicofol | 7.14 | 1.43 |
| dicrotophos | 34.99 | 6.11 |
| dicyclanil | 85.32 | 10.71 |
| dieldrin | 0.00 | 0.00 |
| dienochlor | 0.00 | 0.00 |
| diethatyl ethyl | 25.21 | 5.73 |
| diethofencarb | 37.73 | 7.68 |
| diethylcarbamazine | 51.59 | 8.80 |
| diethyltoluamide | 12.32 | 1.89 |
| difenacoum | 18.22 | 4.90 |
| difenoconazole | 36.32 | 8.54 |
| difenoxuron | 22.42 | 4.42 |
| difenzoquat | 22.42 | 4.42 |
| difethialone | 16.77 | 4.84 |
| diflubenzuron | 45.27 | 7.94 |
| diflufenican | 37.01 | 7.99 |
| diflumetorim | 37.01 | 7.99 |
| difunon | 37.85 | 6.13 |
| 13,14-dihydro-3,4-epoxy melissodoric acid methyl ester acetate | 32.48 | 9.56 |
| dihydropyrone | 36.26 | 6.39 |
| dikegulac sodium | 76.82 | 6.88 |
| dimefluazole | 39.79 | 7.72 |
| dimefox | 53.32 | 6.32 |
| dimehypo | 80.75 | 20.33 |
| dimepiperate | 30.30 | 5.90 |
| dimethachlon | 33.04 | 4.48 |
| dimethachlor | 19.84 | 3.67 |
| dimethametryn | 39.55 | 7.70 |
| dimethipin | 66.86 | 8.45 |
| dimethirimol | 23.07 | 3.89 |
| dimethoate | 71.18 | 10.88 |
| dimethomorph | 35.72 | 9.08 |
| O-(4,6-dimethoxypyrimidin-2-yl) salicylic acid | 57.22 | 10.13 |
| dimethyl 4-(methylthio)phenylphosphate | 20.58 | 3.71 |
| dimethyl phthalate | 48.91 | 6.56 |
| dimethynonane | 2.25 | 0.61 |
| dimetilan | 38.14 | 6.67 |
| dinactin | 15.72 | 8.78 |
| dinex-diclexine | 36.59 | 6.32 |
| diniconazole-M | 26.89 | 5.72 |
| dinitramine | 48.24 | 9.64 |
| dinitrophenol | 53.60 | 5.72 |
| dinobuton | 36.40 | 8.69 |
| dinocap | 26.79 | 7.08 |
| dinopenton | 36.26 | 8.61 |
| dinoprop | 38.77 | 6.29 |
| dinoseb | 39.17 | 6.28 |
| dinoseb acetate | 37.46 | 7.09 |
| dinoterb | 38.22 | 6.31 |
| dinoterb acetate | 36.73 | 7.13 |
| dinoterbon | 38.84 | 8.37 |
| dioxabenzofos | 50.04 | 6.21 |
| dioxacarb | 78.01 | 11.49 |
| dioxapyrrolomycin | 46.50 | 8.41 |
| dioxathion | 59.17 | 17.52 |
| diphacinone | 26.68 | 5.54 |
| diphenamid | 9.58 | 1.65 |
| diphenane | 39.78 | 5.98 |
| diphenylamine | 12.64 | 1.46 |
| dipropetryn | 34.18 | 6.65 |
| dipropyl pyridine-2,5-dicarboxylate | 50.54 | 9.35 |
| dipyrithione | 100.00 | 14.50 |
| diquat | 100.00 | 5.94 |
| disulfoton | 37.14 | 7.25 |
| ditalimfos | 52.50 | 9.26 |
| dithianon | 75.10 | 12.15 |
| dithiazinine iodide | 70.70 | 6.85 |
| dithiopyr | 42.91 | 10.54 |
| diuron | 30.49 | 4.42 |
| DMCP | 27.91 | 4.22 |
| DNOC | 46.98 | 5.74 |
| dodemorph | 7.22 | 1.78 |
| dodicin | 47.98 | 14.23 |
| dodine | 30.09 | 6.47 |
| doproxydim | 17.87 | 4.94 |
| 3,5-DP | 29.74 | 4.15 |
| drazoxolon | 36.56 | 5.15 |
| DSMA | 83.06 | 10.58 |
| dunnione | 21.21 | 3.28 |
| DuPont oxime fungicide | 49.15 | 9.73 |
| dymanthine | 0.51 | 0.15 |
| dymuron | 28.28 | 5.55 |
| echinocandin | 58.51 | 46.39 |
| econazole | 32.90 | 7.35 |
| edifenphos | 25.62 | 4.93 |
| eglinazine | 85.90 | 12.80 |
| emamectin benzoate | 42.38 | 28.99 |
| embelin | 33.80 | 7.85 |
| empenthrin | 9.26 | 2.20 |
| endosulfan | 32.14 | 6.08 |
| endothall | 77.62 | 9.30 |
| endothion | 62.95 | 11.22 |
| endrin | 0.00 | 0.00 |
| EPBP | 18.66 | 3.73 |
| 5-epi-30-hydroxymilbemycin a14 | 41.16 | 19.81 |
| EPN | 57.83 | 11.62 |
| epoxiconazole | 36.14 | 6.78 |
| EPTC | 25.02 | 4.06 |
| erbon | 23.26 | 4.72 |
| esfenvalerate | 19.68 | 5.41 |
| esprocarb | 16.63 | 3.46 |
| etacelasil | 42.30 | 10.05 |
| etaconazole | 46.33 | 9.25 |
| ethalfluralin | 23.38 | 4.92 |
| ethametsulfuron methyl | 68.58 | 17.39 |
| ethephon | 65.16 | 5.91 |
| ethidimuron | 69.02 | 11.63 |
| ethiofencarb | 41.94 | 6.79 |
| ethiolate | 22.65 | 3.07 |
| ethion | 52.42 | 13.37 |
| ethiozin | 51.12 | 7.19 |
| ethirimol | 38.99 | 6.51 |
| ethoate methyl | 60.80 | 10.10 |
| ethofumesate | 32.67 | 6.19 |
| ethoprop | 30.44 | 5.52 |
| ethoxyquin | 15.08 | 2.51 |
| ethoxysulfuron | 56.36 | 14.37 |
| ethychlozate | 57.24 | 7.73 |
| ethylan | 0.00 | 0.00 |
| ethylene | 0.00 | 0.00 |
| ethylene bis trichloroacetate | 39.79 | 7.40 |
| ethylene dibromide | 0.00 | 0.00 |
| ethylene dichloride | 0.00 | 0.00 |
| 2-ethyl-1,3-hexanediol | 31.17 | 4.31 |
| ethylidene diacetate | 45.47 | 5.25 |
| etinofen | 54.39 | 7.80 |
| etofenprox | 3.82 | 1.03 |
| etridiazole | 32.03 | 4.30 |
| etrimfos | 42.40 | 8.32 |
| eugenol (clove oil) | 10.84 | 1.35 |
| exalamide | 40.49 | 7.01 |
| EXD | 55.81 | 8.92 |
| famoxadone | 32.77 | 7.56 |
| famphur | 52.62 | 10.73 |
| fenaminosulf | 49.95 | 8.77 |
| fenamiphos | 28.66 | 6.24 |
| fenapanil | 35.81 | 6.92 |
| fenarimol | 32.92 | 6.34 |
| fenazaflor | 50.62 | 9.96 |
| fenazaquin | 18.51 | 4.02 |
| fenbuconazole | 34.51 | 7.60 |
| fenchlorazole | 31.89 | 5.98 |
| fenchlorphos | 23.05 | 3.96 |
| fenclorim | 19.28 | 2.31 |
| fenfuram | 57.34 | 7.69 |
| fenitropan | 52.58 | 10.22 |
| fenitrothion | 37.87 | 6.51 |
| fenobucarb (BPMC) | 40.47 | 6.51 |
| fenoprop | 28.18 | 4.28 |
| fenothiocarb | 19.17 | 3.63 |
| fenoxaprop ethyl | 23.78 | 5.22 |
| fenoxaprop-P-ethyl | 23.91 | 5.25 |
| fenoxycarb | 38.07 | 7.92 |
| fenpiclonil | 43.32 | 5.72 |
| fenpropathrin | 21.55 | 5.36 |
| fenpropidin | 0.46 | 0.11 |
| fenpropimorph | 6.69 | 1.69 |
| fenpyroximate | 21.70 | 6.27 |
| fenridazon | 41.86 | 6.44 |
| fenson | 36.02 | 5.41 |
| fensulam | 60.58 | 15.52 |
| fensulfothion | 32.14 | 6.43 |
| fenthiaprop ethyl | 15.97 | 3.61 |
| fenthion | 24.78 | 4.48 |
| fentiazon | 39.07 | 6.92 |
| fenticonazole | 26.79 | 7.28 |
| fentrifanil | 35.82 | 7.28 |
| fenuron | 36.04 | 4.42 |
| fenvalerate | 19.69 | 5.41 |
| ferbam | 48.18 | 12.19 |
| ferimzone | 28.78 | 5.43 |
| filipin III | 46.26 | 24.59 |
| flamprop-isopropyl | 21.57 | 5.12 |
| flamprop-methyl | 24.31 | 5.12 |
| flazasulfuron | 60.18 | 13.64 |
| flocoumafen | 17.11 | 5.44 |
| fluazifop butyl | 19.57 | 4.85 |
| fluazifop-P-butyl | 19.57 | 4.85 |
| fluazifop methyl | 19.98 | 4.16 |
| fluazinam | 43.80 | 9.98 |
| flubenzimine | 26.22 | 5.85 |
| fluchloralin | 23.15 | 4.92 |
| flucofuron | 33.20 | 7.08 |
| fluconazole | 67.47 | 12.39 |
| flucycloxuron | 32.72 | 9.52 |
| flucythrinate | 28.92 | 8.47 |
| flucytosine | 100.00 | 7.61 |
| fludioxinil | 77.01 | 10.22 |
| fluenetil | 32.29 | 5.55 |
| flufenoxuron | 29.56 | 7.65 |
| flufenzine | 26.23 | 4.13 |
| flumetover | 7.91 | 1.89 |
| flumetralin | 20.84 | 4.94 |
| flumiclorac pentyl | 44.80 | 11.56 |
| fluometuron | 29.83 | 4.42 |
| fluorbenside | 0.00 | 0.00 |
| fluorchloridone | 11.74 | 2.01 |
| fluorglycofen ethyl | 37.55 | 9.60 |
| fluoroacetamide | 100.00 | 5.77 |
| fluorodifen | 27.71 | 4.92 |
| fluoromide | 31.92 | 4.28 |
| fluoronitrofen | 15.41 | 2.46 |
| fluothiuron | 24.56 | 4.46 |
| fluotrimazole | 17.22 | 3.88 |
| flupropacil | 37.25 | 8.60 |
| flupropadine | 1.87 | 0.49 |
| fluquinconazole | 47.56 | 9.27 |
| flurazole | 39.53 | 6.82 |
| flurenol-butyl | 27.90 | 5.31 |
| fluridone | 22.01 | 4.27 |
| fluroxpyr | 74.14 | 10.29 |
| flurprimidol | 34.52 | 6.83 |
| flurtamone | 36.12 | 7.14 |
| flusilazole | 20.09 | 3.85 |
| flusulfamide | 44.27 | 9.32 |
| flutolanil | 29.70 | 6.09 |
| flutriafol | 40.14 | 7.35 |
| fluvalinate | 25.79 | 8.02 |
| fluxofenim | 43.40 | 7.77 |
| folcysteine | 81.97 | 9.53 |
| folpet | 45.71 | 6.78 |
| fomesafen | 47.78 | 11.18 |
| fonofos | 25.22 | 4.18 |
| forchlorfenuron | 64.98 | 9.74 |
| formetanate HCl | 79.04 | 6.00 |
| formothion | 47.31 | 7.90 |
| fosamine ammonium | 74.80 | 8.98 |
| fosetyl aluminum | 61.98 | 14.80 |
| fosmethilan | 35.21 | 8.32 |
| fospirate | 27.54 | 4.54 |
| fosthiazate | 56.85 | 10.85 |
| fosthietan | 72.54 | 11.12 |
| fthalide (phthalide) | 33.24 | 4.28 |
| fuberidazole | 74.60 | 7.82 |
| fungichromin | 50.42 | 27.28 |
| furalaxyl | 34.32 | 7.23 |
| furathiocarb | 35.89 | 9.61 |
| furconazole-cis | 43.43 | 9.70 |
| furmecyclox | 12.64 | 2.34 |
| genaconazole | 49.94 | 10.12 |
| genite | 33.52 | 5.41 |
| gibberellic acid | 42.35 | 9.43 |
| glufosinate ammonium | 100.00 | 7.88 |
| glyceollin | 29.76 | 6.30 |
| glyodin | 49.59 | 5.65 |
| glyoxime | 85.46 | 5.42 |
| glyphosate | 100.00 | 11.37 |
| glyphosine | 100.00 | 16.57 |
| griseofulvin | 29.15 | 6.44 |
| guazatine | 58.09 | 20.29 |
| hadacidin | 100.00 | 8.61 |
| halethazole | 11.03 | 2.57 |
| halfenprox | 5.73 | 1.61 |
| halicrinate | 27.98 | 4.49 |
| halofenozide | 35.68 | 7.92 |
| halosulfuron methyl | 61.71 | 15.79 |
| haloxyfop methyl | 15.06 | 3.30 |
| harzianopyridone | 36.88 | 7.36 |
| heptachlor | 0.00 | 0.00 |
| heptenophos | 26.16 | 3.97 |
| heptopargil | 4.17 | 0.71 |
| hexachloroacetone | 8.73 | 1.11 |
| hexachlorophene | 7.99 | 1.62 |
| hexaconazole | 32.66 | 6.65 |
| hexaflumuron | 57.28 | 13.63 |
| hexaflurate | 100.00 | 6.27 |
| hexazinone | 29.13 | 5.41 |
| hexetidine | 25.43 | 8.01 |
| hexylresorcinol | 18.19 | 2.83 |
| hexythiazox | 36.29 | 8.21 |
| HW 863 | 23.26 | 5.42 |
| hycanthone | 52.16 | 12.51 |
| hydantocidin | 100.00 | 13.45 |
| hydramethylnon | 26.46 | 8.27 |
| hydrated copper sulfate | 100.00 | 2.50 |
| 2-hydrazinoethanol | 100.00 | 7.04 |
| hydrogen cyanide | 100.00 | 3.10 |
| hydroprene | 9.23 | 2.26 |
| hymexazol | 11.53 | 0.80 |
| ICI 153066 | 34.74 | 6.95 |
| imazalil | 38.57 | 7.22 |
| imazamethabenz-methyl (meta) | 37.96 | 7.89 |
| imazamethabenz-methyl (para) | 37.96 | 7.89 |
| imazapyr | 47.41 | 6.47 |
| imazaquin | 46.42 | 9.51 |
| imazethapyr | 47.55 | 9.79 |
| imazosulfuron | 59.00 | 13.58 |
| imibenconazole | 33.11 | 7.63 |
| imidacloprid | 70.62 | 10.87 |
| iminoctadine | 51.67 | 16.21 |
| imiprothrin | 46.61 | 11.03 |
| inabenfide | 55.59 | 11.45 |
| indole-3-butyric acid | 62.92 | 8.78 |
| inezin | 25.48 | 4.90 |
| iodofenphos | 22.44 | 4.06 |
| iodopropargyl fungicide | 9.40 | 1.58 |
| ioxynil | 25.21 | 3.08 |
| ipazine | 34.98 | 6.31 |
| ipconazole | 29.29 | 6.79 |
| iprobenfos | 22.56 | 4.66 |
| iprodione | 42.20 | 8.36 |
| IPSP | 33.31 | 7.31 |
| isazophos | 33.50 | 6.91 |
| Ishihara pyridyl carbamate fungicide | 43.87 | 7.47 |
| isobenzan | 3.38 | 0.62 |
| isocarbamid | 66.15 | 9.34 |
| isoconazole | 31.26 | 7.35 |
| isofenphos | 44.60 | 10.96 |
| isomethiozin | 32.24 | 6.11 |
| isopamphos | 67.08 | 6.22 |
| isoprocarb | 67.08 | 6.22 |
| isopropalin | 38.54 | 5.68 |
| isopropylate | 21.39 | 4.92 |
| N-(isopropyloxycarbonyl)-L-valine-(R)-1-(B-naphthyl)ethylamide | 37.16 | 5.98 |
| isoprothiolane | 28.57 | 7.52 |
| isoprothiolane | 39.00 | 7.97 |
| isoproturon | 27.13 | 4.42 |
| isothioate | 34.26 | 6.24 |
| isouron | 17.97 | 3.03 |
| isoxaben | 20.95 | 5.13 |
| isoxaflutole | 23.97 | 4.84 |
| isoxapyrifop | 15.37 | 3.51 |
| isoxathion | 20.86 | 4.15 |
| itraconazole | 41.72 | 18.28 |
| ivermectin B1a | 40.26 | 25.98 |
| ivermectin B1a | 40.26 | 25.98 |
| ivermectin B1b | 41.11 | 25.98 |
| japonilure | 16.62 | 3.17 |
| kadethrin | 31.85 | 8.46 |
| kainic acid | 70.50 | 11.44 |
| karbutilate | 42.33 | 8.92 |
| kasugamycin | 91.91 | 24.00 |
| kayaphenon | 8.34 | 1.41 |
| ketoconazole | 52.10 | 17.07 |
| kinoprene | 19.68 | 4.66 |
| knockbal | 39.59 | 6.48 |
| a-kosin | 24.79 | 8.27 |
| b-kosin | 25.78 | 8.59 |
| kresoxim-methyl | 21.95 | 4.76 |
| lactofen | 30.85 | 8.32 |
| laetisaric acid | 24.16 | 6.02 |
| lanoconazole | 56.78 | 10.46 |
| lardolure | 13.43 | 3.03 |
| lenacil | 37.94 | 5.97 |
| leptophos | 24.35 | 4.59 |
| leucanicidin | 28.16 | 17.00 |
| lindane | 0.00 | 0.00 |
| linuron | 33.18 | 5.01 |
| loflucarban | 45.48 | 7.97 |
| lucanthone hydrochloride | 64.68 | 4.68 |
| lucensomycin | 59.52 | 30.74 |
| lythidathion | 58.68 | 11.27 |
| macbal | 42.34 | 5.68 |
| magnesium phosphide | 100.00 | 3.22 |
| malathion | 49.70 | 11.33 |
| maleic hydrazide | 100.00 | 6.82 |
| mazidox | 100.00 | 13.68 |
| S-MBTU | 34.30 | 6.10 |
| MCPA | 38.79 | 5.02 |
| MCPB | 30.83 | 4.81 |
| MCPCA | 30.95 | 5.82 |
| mebendazole | 61.69 | 10.79 |
| mebenil | 31.86 | 4.60 |
| mecarbam | 54.50 | 12.53 |
| mecarphon | 56.46 | 10.28 |
| mecoprop | 29.76 | 4.25 |
| medinoterb acetate | 34.24 | 7.12 |
| mefenacet | 27.65 | 5.06 |
| mefluidide | 61.58 | 11.52 |
| MEMC | 7.05 | 0.60 |
| menazon | 74.38 | 12.79 |
| mepanipyrim | 38.25 | 5.97 |
| mepartricin A | 54.89 | 46.47 |
| mepartricin B | 58.64 | 48.77 |
| mephosfolan | 56.31 | 10.22 |
| mepronil | 25.01 | 4.83 |
| merphos | 21.75 | 5.11 |
| metalaxyl | 30.64 | 6.49 |
| metaldehyde | 39.43 | 5.49 |
| metam | 100.00 | 7.60 |
| metamitron | 50.64 | 6.59 |
| metam sodium | 100.00 | 10.39 |
| metazachlor | 40.82 | 7.59 |
| metconazole | 32.46 | 6.68 |
| methabenzthiazuron | 53.30 | 7.46 |
| methacrifos | 47.51 | 7.88 |
| methamidophos | 85.74 | 8.32 |
| metham sodium | 100.00 | 5.68 |
| methasulfocarb | 76.28 | 12.19 |
| methazole | 25.65 | 3.65 |
| methfuroxam | 31.78 | 5.08 |
| methidathion | 63.40 | 11.46 |
| methiocarb | 35.07 | 5.71 |
| methiquat chloride | 100.00 | 6.76 |
| methiuron | 36.06 | 5.19 |
| methometon | 52.76 | 11.51 |
| methomyl | 50.33 | 6.13 |
| methoprene | 10.23 | 2.86 |
| methoprotryne | 46.41 | 9.40 |
| methoxychlor | 0.00 | 0.00 |
| methoxyfenozide | 20.43 | 5.57 |
| methyl anthranilate | 74.91 | 7.94 |
| methyl bromide | 0.00 | 0.00 |
| methyl 2-(2-chloro-4-fluoro-5[(3-oxoperhydro[1,2,4]thiadiazolo[3,4-a]pyridazin-1-ylidene)amino]phenylthio)acetate | 45.94 | 10.73 |
| methyl-3,6-dichloro-o-anisate | 17.65 | 2.49 |
| methyldymron | 28.23 | 5.53 |
| 1,1'-methylenedi(thiosemicarbazide) | 100.00 | 13.43 |
| methyl eugenol | 0.00 | 0.00 |
| methyl metiram | 86.06 | 52.16 |
| 3-(3-methyl-1,2,4-oxadiazole)azabicyclo[2.2.1]heptane | 17.76 | 2.15 |
| methyl parathion | 43.71 | 6.95 |
| metiram | 100.00 | 55.58 |
| metobromuron | 34.89 | 5.01 |
| metolachlor | 12.62 | 2.67 |
| metolcarb | 47.07 | 5.68 |
| metoxuron | 29.88 | 4.63 |
| metribuzin | 45.93 | 7.18 |
| metsulfovax | 46.35 | 7.03 |
| metsulfuron-methyl | 62.31 | 14.34 |
| mevinphos | 42.94 | 6.91 |
| mexacarbate | 33.06 | 5.68 |
| miconazole | 30.88 | 7.16 |
| mildiomycin | 85.13 | 31.14 |
| milneb | 64.19 | 15.12 |
| mirex | 0.00 | 0.00 |
| molinate | 40.53 | 5.98 |
| monalide | 40.53 | 5.98 |
| monisouron | 43.79 | 6.76 |
| monocrotophos | 59.90 | 9.73 |
| monolinuron | 37.48 | 5.43 |
| monuron | 33.04 | 4.42 |
| morphothion | 76.64 | 14.49 |
| MSMA | 77.99 | 7.65 |
| mulundocandin | 63.03 | 47.39 |
| muscalure | 0.00 | 0.00 |
| myclobutanil | 37.45 | 7.55 |
| myclozolin | 35.65 | 6.37 |
| myxothiazol A | 27.87 | 9.55 |
| NAA | 38.28 | 4.66 |
| nabam | 100.00 | 18.71 |
| naftifine | 6.53 | 1.24 |
| naled | 22.27 | 3.75 |
| naphthalene | 0.00 | 0.00 |
| naphthalene | 0.00 | 0.00 |
| naphthaleneacetamide | 39.07 | 4.81 |
| naphthalic anhydride | 42.27 | 4.60 |
| 2-naphthol | 15.78 | 1.42 |
| naproanilide | 28.90 | 5.48 |
| napropamide | 10.73 | 2.13 |
| naptalam | 51.07 | 9.10 |
| napthalene acetamide | 39.07 | 4.81 |
| (2-napthyloxy)acetic acid | 42.02 | 5.44 |
| natamycin | 64.43 | 30.68 |
| nearal | 15.10 | 2.32 |
| neburon | 35.96 | 6.65 |
| neo-asozin | 76.63 | 6.01 |
| neryl formate | 76.63 | 6.01 |
| niclosamide | 21.77 | 3.53 |
| niclosamide | 50.72 | 9.11 |
| nicosulfuron | 62.88 | 16.01 |
| nicotine | 20.29 | 2.45 |
| nifluridide | 81.21 | 15.01 |
| nifuratel | 75.67 | 13.20 |
| nikkomycin X | 90.86 | 27.24 |
| nipyraclofen | 42.31 | 7.22 |
| nitenpyram | 63.64 | 11.60 |
| nithiazine | 100.00 | 10.58 |
| nitralin | 44.23 | 10.18 |
| nitrofen | 15.86 | 2.46 |
| nitrothal isopropyl | 42.89 | 8.93 |
| norea | 29.27 | 4.89 |
| norflurazon | 35.54 | 5.87 |
| novaluron | 67.32 | 17.26 |
| nuarimol | 33.93 | 6.30 |
| Nuclear Polyhedrosis Virus | 33.93 | 6.30 |
| nystatin | 53.59 | 37.59 |
| n-octyl isothiazalone | 29.27 | 4.86 |
| 2-(octylthio)ethanol | 27.48 | 4.69 |
| ofurace | 36.82 | 6.89 |
| okaramine A | 38.90 | 12.72 |
| oligomycin D | 27.18 | 16.69 |
| omethoate | 70.00 | 10.29 |
| omoconazole | 26.09 | 6.55 |
| omphalotin | 41.84 | 43.92 |
| orbencarb | 19.33 | 3.46 |
| oryzalin | 54.60 | 12.35 |
| oudemansin A | 20.01 | 4.44 |
| oxabetrinil | 66.00 | 10.33 |
| oxadiargyl | 25.12 | 5.48 |
| oxadiazon | 18.26 | 4.09 |
| oxadixyl | 42.65 | 8.46 |
| oxamniquine | 71.03 | 14.24 |
| oxamyl | 59.86 | 9.68 |
| oxantel | 41.46 | 6.45 |
| oxiconazole nitrate | 65.61 | 5.67 |
| oxolinic acid | 70.15 | 10.97 |
| oxycarboxin | 67.45 | 11.47 |
| oxydemeton methyl | 47.85 | 8.07 |
| oxydeprofos | 36.86 | 6.71 |
| oxydisulfoton | 48.17 | 9.70 |
| oxyfluorfen | 12.10 | 2.46 |
| oxythioquinox | 57.07 | 7.16 |
| paclobutrazol | 28.57 | 5.84 |
| papulacandin B | 47.26 | 29.92 |
| paraherquamide | 35.66 | 11.70 |
| paraoxon | 41.22 | 7.46 |
| paraquat | 100.00 | 10.00 |
| parathion | 37.61 | 7.03 |
| parinol | 32.65 | 6.28 |
| partricin A | 56.32 | 46.81 |
| partricin B | 60.26 | 49.24 |
| PCNB | 18.16 | 2.46 |
| pebulate | 23.12 | 4.06 |
| pecilocin | 36.13 | 8.41 |
| pefurazoate | 50.80 | 12.62 |
| pelletierine | 56.39 | 6.83 |
| penconazole | 28.84 | 5.24 |
| pencycuron | 31.22 | 6.80 |
| pendimethalin | 35.64 | 7.23 |
| pentachlorobenzoic acid | 23.82 | 3.32 |
| pentachlorophenol | 6.45 | 0.80 |
| pentachlorophenol (PCP) | 6.41 | 0.79 |
| pentanochlor (solan) | 24.37 | 4.31 |
| pepstatin A | 49.85 | 27.35 |
| perfluidone | 58.95 | 12.24 |
| perimycin A | 49.64 | 41.95 |
| permethrin | 14.24 | 3.57 |
| PF 1022A | 33.61 | 24.15 |
| phenamiphos | 28.61 | 6.28 |
| phencapton | 32.62 | 7.27 |
| phenisopham | 37.91 | 9.07 |
| phenmedipham | 61.53 | 12.20 |
| phenothiol | 30.17 | 4.92 |
| d-phenothrin | 14.01 | 3.57 |
| phenthoate | 38.01 | 7.96 |
| phenylmercuric acetate | 35.32 | 3.90 |
| phorate | 48.12 | 8.82 |
| phosalone | 55.70 | 12.12 |
| phosdiphen | 15.15 | 3.38 |
| phosfolan | 63.57 | 10.57 |
| phosmet | 62.93 | 11.69 |
| phosphamidon | 37.18 | 8.06 |
| phoxim | 38.05 | 7.63 |
| picloram | 71.11 | 8.84 |
| pimaricin | 58.42 | 27.73 |
| pindone | 32.35 | 5.16 |
| piperalin | 22.11 | 4.90 |
| piperazine | 100.00 | 7.65 |
| piperazine adipate | 91.96 | 8.64 |
| piperazine tartrate | 100.00 | 9.17 |
| piperonyl butoxide | 40.37 | 10.51 |
| piperophos | 36.59 | 9.41 |
| piproctanylium bromide | 71.37 | 7.12 |
| pirimicarb | 26.57 | 4.83 |
| pirimiphos ethyl | 31.64 | 7.58 |
| pirimiphos methyl | 35.56 | 7.56 |
| pisiferic acid | 18.55 | 4.43 |
| plifenate | 12.56 | 2.30 |
| PMA | 35.61 | 3.95 |
| pneumocandin A0 | 61.11 | 49.94 |
| pneumocandin B0 | 66.42 | 52.50 |
| polyoxin B | 100.00 | 32.56 |
| polyoxin D | 100.00 | 32.79 |
| potassium iodide | 100.00 | 6.67 |
| prallethrin | 16.68 | 3.99 |
| praziquantel | 35.87 | 7.62 |
| precocene 1 | 0.00 | 0.00 |
| pretilachlor | 19.40 | 4.63 |
| primisulfuron methyl | 79.31 | 20.51 |
| probenazole | 42.43 | 5.67 |
| prochloraz | 42.75 | 9.83 |
| procymidone | 22.01 | 3.67 |
| prodiamine | 42.35 | 9.53 |
| profenofos | 22.67 | 4.70 |
| profluralin | 22.78 | 4.94 |
| proglinazine | 71.07 | 11.48 |
| prohexadione | 77.02 | 11.48 |
| promecarb | 35.31 | 5.68 |
| promecarb A | 23.58 | 5.11 |
| prometon | 33.79 | 5.89 |
| prometryn | 36.73 | 6.66 |
| pronamide | 15.84 | 2.68 |
| propachlor | 10.66 | 1.61 |
| propamocarb | 47.54 | 7.92 |
| propanil | 35.09 | 4.71 |
| propaphos | 22.71 | 4.87 |
| propaquizafop | 32.20 | 9.16 |
| propargite | 26.45 | 6.75 |
| propazine | 43.08 | 7.08 |
| propetamphos | 50.05 | 10.59 |
| propham | 37.65 | 5.02 |
| propiconazole | 43.18 | 9.08 |
| propineb | 85.08 | 12.06 |
| propionic acid | 67.66 | 4.40 |
| propisochlor | 20.10 | 4.29 |
| propoxur | 46.73 | 7.28 |
| propyl isome | 35.48 | 9.03 |
| prosulfocarb | 22.91 | 4.46 |
| prosulfuron | 46.35 | 11.38 |
| prothiocarb hydrochloride | 72.58 | 4.88 |
| prothiophos | 23.10 | 4.81 |
| prothoate | 50.17 | 10.34 |
| proxan | 51.72 | 5.22 |
| prynachlor | 10.47 | 1.64 |
| pymetrozine | 74.89 | 10.55 |
| pyracarbolid | 37.14 | 5.71 |
| pyraclofos | 33.91 | 7.63 |
| pyrantel | 35.25 | 5.04 |
| pyrazolate | 27.15 | 6.78 |
| pyrazophos | 48.29 | 11.81 |
| pyrazosulfuron ethyl | 63.45 | 16.18 |
| pyrazoxyfen | 28.95 | 6.99 |
| pyrethrin I | 15.09 | 3.98 |
| pyributicarb | 28.25 | 6.66 |
| pyrichlor | 12.64 | 1.25 |
| pyridaben | 15.82 | 4.07 |
| pyridaphenthion | 43.04 | 9.15 |
| pyridate | 28.47 | 7.36 |
| pyridinitnil | 3.20 | 0.46 |
| pyrifenox | 13.30 | 2.40 |
| pyrimethanil | 43.51 | 5.96 |
| pyrimidifen | 30.56 | 8.30 |
| pyriproxyfen | 14.23 | 3.02 |
| pyrithione | 14.69 | 1.17 |
| pyroquilon | 14.55 | 1.64 |
| pyroxychlor | 11.22 | 1.57 |
| pyroxyfur | 17.09 | 2.95 |
| pyrrolnitrin | 43.67 | 5.76 |
| pyrvinium pamoate | 40.98 | 9.64 |
| quercimeritrin | 87.67 | 24.58 |
| quinacrine | 16.33 | 4.57 |
| quinalphos | 40.50 | 7.56 |
| quinazamid | 95.86 | 10.41 |
| quinclorac | 36.83 | 4.73 |
| quinmerac | 36.02 | 4.73 |
| 8-quinolinol | 39.68 | 3.59 |
| quinoxyfen | 8.73 | 1.41 |
| quizalofop-ethyl | 27.41 | 6.33 |
| resmethrin | 21.42 | 5.37 |
| rhodethanil | 27.21 | 3.77 |
| rotenone | 17.09 | 4.27 |
| ryanodine | 17.05 | 4.25 |
| salclomide | 41.85 | 9.06 |
| salicylanilide | 52.00 | 7.22 |
| a-santoin | 20.26 | 3.53 |
| saperconazole | 37.74 | 15.94 |
| schradan | 100.00 | 16.66 |
| scilliroside | 50.26 | 21.19 |
| secbumeton | 40.86 | 7.12 |
| sertaconazole | 29.93 | 7.22 |
| sesone | 53.74 | 10.01 |
| sethoxydim | 16.90 | 4.35 |
| siccanin | 10.83 | 2.80 |
| siduron | 36.21 | 6.27 |
| simazine | 69.38 | 9.54 |
| simetryn | 58.97 | 9.12 |
| sindone | 4.58 | 1.11 |
| sinefungin | 100.00 | 25.17 |
| sinigrin | 79.65 | 16.79 |
| sintofen | 42.49 | 9.43 |
| sodiumfluoaluminate | 100.00 | 4.10 |
| sodium fluoride | 100.00 | 4.70 |
| sodium fluoroacetate | 100.00 | 8.58 |
| sodium hexafluorosilicate | 80.00 | 2.26 |
| sodium pentachlorophenate | 31.66 | 5.29 |
| sodium propionate | 64.01 | 6.10 |
| sodium tetrathiocarbonate | 100.00 | 13.70 |
| soraphen A | 29.31 | 11.45 |
| spinosyn | 22.45 | 11.98 |
| spiroxamine | 19.59 | 5.03 |
| stibocaptate | 100.00 | 51.02 |
| stibophen | 100.00 | 22.46 |
| stilbazium iodide | 64.37 | 6.22 |
| strobilurin A | 21.91 | 4.31 |
| strychnine | 26.47 | 5.36 |
| sulbentine | 48.81 | 9.90 |
| sulcofuron | 45.19 | 12.24 |
| sulconazole | 30.48 | 6.98 |
| sulcotrione | 42.64 | 8.30 |
| sulfentrazone | 64.06 | 11.38 |
| sulfluramid | 78.48 | 19.93 |
| sulfometuron-methyl | 60.92 | 13.61 |
| sulfotep | 45.07 | 9.93 |
| sulfoxide | 26.12 | 6.51 |
| sulfur | 0.00 | 0.00 |
| sulfuryl fluroide | 100.00 | 4.91 |
| sulprofos | 24.68 | 5.33 |
| suramin sodium | 90.43 | 79.28 |
| swep | 45.27 | 6.45 |
| 2,4,5-T | 35.75 | 4.91 |
| 2,3,6-TBA | 28.36 | 3.32 |
| TCA | 35.51 | 3.08 |
| TDE | 0.00 | 0.00 |
| tebuconazole | 30.80 | 6.82 |
| tebufenozide | 41.48 | 8.72 |
| tebufenpyrad | 24.98 | 5.96 |
| tebuthiuron | 46.82 | 8.02 |
| tecnazene | 19.60 | 2.47 |
| teflubenzuron (CME-13406) | 38.95 | 7.65 |
| tefluthrin | 14.75 | 3.58 |
| temephos | 32.47 | 9.03 |
| tenonitrozole | 73.81 | 10.26 |
| TEPA | 100.00 | 12.15 |
| TEPP | 41.73 | 8.50 |
| terbacil | 31.72 | 4.72 |
| terbinafine | 0.40 | 0.09 |
| terbucarb | 22.56 | 5.26 |
| terbufos | 34.59 | 7.46 |
| terbumeton | 39.50 | 6.94 |
| terbuthylazine | 49.08 | 8.12 |
| terbutryn | 42.17 | 7.70 |
| terconazole | 42.57 | 14.67 |
| tetrachloroethylene | 0.00 | 0.00 |
| tetrachlorothiophene | 0.00 | 0.00 |
| tetrachlorvinphos | 23.60 | 4.76 |
| tetraconazole | 61.13 | 12.61 |
| tetradifon | 15.98 | 2.83 |
| tetrafluoron | 63.87 | 11.11 |
| tetrahydroechinocandin B | 57.96 | 46.63 |
| tetramethrin | 31.63 | 7.50 |
| tetramisole | 61.44 | 7.75 |
| tetranactin | 15.04 | 8.78 |
| tetrasul | 0.00 | 0.00 |
| thallium sulfate | 100.00 | 6.81 |
| thanite | 26.60 | 5.05 |
| thaxtomin A | 64.75 | 17.88 |
| thenylchlor | 9.92 | 2.09 |
| thiabendazole | 77.54 | 8.51 |
| thiabendazole | 77.54 | 8.51 |
| thiangazole | 52.21 | 18.75 |
| thiazafluron | 74.74 | 9.41 |
| thiazopyr | 50.25 | 12.06 |
| thicyofen | 49.81 | 7.18 |
| thidiazuron | 73.40 | 9.54 |
| thifensulfuron-methyl | 70.24 | 15.72 |
| thifluzamide | 30.49 | 6.93 |
| thiobencarb | 19.33 | 3.46 |
| thiocyclam hydrogenoxalate | 79.99 | 6.70 |
| thiodicarb | 38.90 | 9.50 |
| thiofanox | 34.42 | 6.09 |
| thiometon | 36.99 | 6.24 |
| thionazin | 58.52 | 9.87 |
| thiophanate | 81.24 | 19.98 |
| thiophanate methyl | 85.56 | 18.81 |
| thioquinox | 23.47 | 2.78 |
| thiram | 46.82 | 7.71 |
| thymol | 11.71 | 1.42 |
| thymyl N-isoamylcarbamate | 23.77 | 5.11 |
| TIBA | 23.39 | 3.32 |
| tiocarbazil | 13.85 | 3.43 |
| tioconazole | 33.28 | 7.24 |
| toclofos methyl | 23.41 | 4.11 |
| tolciclate | 14.52 | 3.09 |
| tolindate | 15.16 | 3.02 |
| tolnaftate | 17.33 | 3.44 |
| tolyfluanid | 22.93 | 4.63 |
| tralkoxydim | 21.29 | 4.45 |
| tralomethrin | 17.52 | 5.38 |
| transfluthrin | 17.01 | 3.62 |
| triacetin | 62.13 | 10.27 |
| triadimefon | 36.47 | 7.15 |
| triadimenol | 36.16 | 7.32 |
| triallate | 17.28 | 3.52 |
| triapenthenol | 26.55 | 5.52 |
| triarimol | 33.07 | 6.40 |
| triasulfuron | 56.96 | 13.59 |
| triazamate | 36.26 | 8.10 |
| triazophos | 38.95 | 7.74 |
| triazoxide | 83.11 | 10.98 |
| tribonate | 47.17 | 12.33 |
| tribufos | 24.59 | 5.98 |
| tricamba | 33.16 | 4.59 |
| trichlamide | 41.78 | 9.03 |
| trichlorfon | 33.37 | 5.01 |
| 2,3,6-trichlorobenzoic acid | 23.39 | 3.32 |
| trichloronate | 25.66 | 4.25 |
| triclopyr | 44.92 | 6.16 |
| tricyclazole | 46.38 | 4.98 |
| tridemorph | 8.00 | 2.18 |
| tridiphane | 12.17 | 2.00 |
| trietazine | 49.40 | 8.77 |
| trifenmorph | 28.53 | 6.42 |
| triflumazid | 29.43 | 6.80 |
| triflumizole | 36.00 | 7.61 |
| triflumuron | 46.17 | 9.26 |
| trifluralin | 22.85 | 4.92 |
| triflusulfuron | 63.16 | 17.40 |
| triforine | 55.48 | 13.95 |
| trimethacarb (H variant) | 42.34 | 5.68 |
| trinactin | 15.38 | 8.78 |
| trinexapac | 78.33 | 11.50 |
| triphenyl tin acetate | 9.50 | 2.02 |
| triphenyl tin hydroxide | 6.25 | 1.14 |
| triprene | 13.18 | 3.58 |
| triticonazole (RPA 400727) | 29.18 | 6.74 |
| TSMTB | 28.43 | 3.86 |
| tubericidin | 100.00 | 16.57 |
| ujothion | 72.27 | 13.12 |
| undecylenic acid | 26.38 | 4.40 |
| uniconazole-P | 29.21 | 5.97 |
| uniconazole-P | 28.35 | 5.71 |
| urea stibamine | 97.20 | 15.24 |
| validamycin | 100.00 | 34.09 |
| validoxylamine A | 100.00 | 23.81 |
| vamidothion | 63.28 | 12.89 |
| veratridine | 46.50 | 22.08 |
| vernolate | 23.19 | 4.08 |
| Verticillium lecanii | 23.19 | 4.08 |
| vinclozolin | 26.31 | 4.41 |
| viridin | 51.38 | 10.81 |
| warfarin | 30.86 | 6.12 |
| xylylcarb | 42.30 | 5.71 |
| 3,5-xylyl methylcarbamate | 42.31 | 5.71 |
| zarilamide | 45.34 | 7.59 |
| zeatin | 74.25 | 10.97 |
| zwittermicin A | 100.00 | 29.85 |

| Table 2-3. SOLVENTS |  |  |  |
| --- | --- | --- | --- |
| Name | Percent hydrophilic surface area | Hydrophilic surface area (cm2/mol x 109) |  |
| acetal | 14.09 | 1.56 |  |
| acetaldehyde | 42.16 | 1.86 |  |
| acetic acid | 73.27 | 3.79 |  |
| acetic anhydride | 48.29 | 3.90 |  |
| acetol | 67.71 | 4.39 |  |
| acetone | 19.38 | 1.11 |  |
| acetone cyanohydrin | 47.19 | 3.77 |  |
| acetonitrile | 27.14 | 1.18 |  |
| acetonylacetone | 22.40 | 2.22 |  |
| acetophenone | 12.46 | 1.11 |  |
| 4-acetylbutyric acid | 55.34 | 5.93 |  |
| acetyl chloride | 29.32 | 1.60 |  |
| 2-acetylcyclohexanone | 24.02 | 2.66 |  |
| 2-acetylcyclopentanone | 27.22 | 2.66 |  |
| acetylene | 0.00 | 0.00 |  |
| 2-acetylpyridine | 38.83 | 3.39 |  |
| 3-acetylpyridine | 47.94 | 4.19 |  |
| 4-acetylpyridine | 44.28 | 3.86 |  |
| 2-acetylthiophene | 13.17 | 1.12 |  |
| acetyltrimethylsilane | 10.79 | 1.11 |  |
| acrolein | 43.82 | 2.31 |  |
| acrylic acid | 84.69 | 5.10 |  |
| acrylonitrile | 44.46 | 2.33 |  |
| allyl acetate | 31.43 | 2.75 |  |
| allyl alcohol | 33.11 | 1.93 |  |
| allylbenzene | 0.00 | 0.00 |  |
| amyl acetate | 23.21 | 2.76 |  |
| aniline | 90.81 | 6.40 |  |
| anisole | 0.00 | 0.00 |  |
| benzaldehyde | 29.69 | 2.26 |  |
| benzene | 0.00 | 0.00 |  |
| benzoic acid | 58.66 | 4.90 |  |
| benzonitrile | 30.12 | 2.28 |  |
| benzophenone | 12.08 | 1.48 |  |
| benzyl acetate | 32.66 | 3.61 |  |
| benzyl alcohol | 34.30 | 2.80 |  |
| benzyl benzoate | 27.05 | 3.86 |  |
| benzyl chloride | 0.00 | 0.00 |  |
| benzyl ether | 0.00 | 0.00 |  |
| biphenyl | 0.00 | 0.00 |  |
| bromobenzene | 0.00 | 0.00 |  |
| bromochloromethane | 0.00 | 0.00 |  |
| bromoethane | 0.00 | 0.00 |  |
| bromoform | 0.00 | 0.00 |  |
| bromomethane | 0.00 | 0.00 |  |
| bromotriflouromethane | 0.00 | 0.00 |  |
| 1,3-butanediol | 51.13 | 4.29 |  |
| n-butane | 0.00 | 0.00 |  |
| 1-butanol | 25.41 | 1.94 |  |
| 2-butanol | 18.74 | 1.43 |  |
| 2-butanone | 15.72 | 1.11 |  |
| 1-butene | 0.00 | 0.00 |  |
| n-butyl acetate | 26.14 | 2.76 |  |
| n-butylamine | 44.64 | 3.51 |  |
| butyl benzyl phthalate | 36.18 | 7.52 |  |
| 1-butyne | 0.00 | 0.00 |  |
| butyraldehyde | 26.31 | 1.86 |  |
| n-butyric acid | 56.18 | 4.41 |  |
| g-butyrolactone | 57.21 | 3.69 |  |
| butyronitrile | 33.31 | 2.34 |  |
| carbon disulfide | 100.00 | 4.63 |  |
| carbon tetrachloride | 0.00 | 0.00 |  |
| chlorobenzene | 0.00 | 0.00 |  |
| 1-chlorobutane | 0.00 | 0.00 |  |
| chlorodifluoromethane | 0.00 | 0.00 |  |
| chloroethane | 0.00 | 0.00 |  |
| chloroform | 0.00 | 0.00 |  |
| chloromethane | 0.00 | 0.00 |  |
| 1-chloropentane | 0.00 | 0.00 |  |
| 1-chloropropane | 0.00 | 0.00 |  |
| o-cresol | 9.76 | 0.80 |  |
| cyclohexane | 0.00 | 0.00 |  |
| cyclohexanol | 16.36 | 1.43 |  |
| cyclohexanone | 18.83 | 1.54 |  |
| cyclohexylamine | 55.45 | 4.97 |  |
| cyclohexyl chloride | 0.00 | 0.00 |  |
| cyclopentane | 0.00 | 0.00 |  |
| cyclopentanol | 19.14 | 1.43 |  |
| cyclopentanone | 22.31 | 1.55 |  |
| DBNPA | 100.00 | 9.83 |  |
| decalin | 0.00 | 0.00 |  |
| n-decane | 0.00 | 0.00 |  |
| 1-decanol | 12.41 | 1.94 |  |
| 3-decanone | 7.37 | 1.11 |  |
| 1,2-dibromoethane | 0.00 | 0.00 |  |
| dibutyl ether | 4.62 | 0.60 |  |
| dibutyl phthalate | 35.49 | 7.60 |  |
| dibutyl sebacate | 19.10 | 5.51 |  |
| dibutyl stearate | 14.43 | 5.51 |  |
| o-dichlorobenzene | 0.00 | 0.00 |  |
| dichlorodiflouromethane | 0.00 | 0.00 |  |
| 1,2-dichloroethane | 0.00 | 0.00 |  |
| 1,2-dichloroethene (trans) | 0.00 | 0.00 |  |
| dichlorofluoromethane | 0.00 | 0.00 |  |
| 1,2-dichlorotetraflouroethane | 0.00 | 0.00 |  |
| diethanolamine | 100.00 | 9.46 |  |
| N,N-diethylacetamide | 16.08 | 1.77 |  |
| diethylamine | 47.08 | 3.74 |  |
| N,N-diethylaniline | 0.00 | 0.00 |  |
| p-diethylbenzene | 0.00 | 0.00 |  |
| diethyl carbonate | 39.59 | 3.97 |  |
| diethylene glycol | 100.00 | 9.18 |  |
| diethylene glycol butyl ether | 57.39 | 8.35 |  |
| diethylene glycol ethyl ether | 70.26 | 8.35 |  |
| diethylene glycol methyl ether | 70.38 | 7.42 |  |
| diethylenetriamine | 100.00 | 9.93 |  |
| diethyl ether | 7.82 | 0.60 |  |
| diethyl fumarate | 48.41 | 6.63 |  |
| diethyl phthalate | 41.29 | 6.65 |  |
| diethyl sulfate | 61.42 | 6.66 |  |
| diethyl sulfide | 0.00 | 0.00 |  |
| diiodomethane | 0.00 | 0.00 |  |
| diisobutyl ketone | 8.08 | 1.11 |  |
| diisopropyl ether | 5.88 | 0.61 |  |
| 1,2-dimethoxybenzene | 0.00 | 0.00 |  |
| N,N-dimethylacetamide | 20.29 | 1.65 |  |
| N,N-dimethylaniline | 0.00 | 0.00 |  |
| N,N-dimethylbenzylamine | 1.05 | 0.12 |  |
| 2,2-dimethyl-1-butanol | 13.61 | 1.43 |  |
| 3,3-dimethyl-1-butanol | 13.61 | 1.43 |  |
| N,N-dimethylethylamine | 1.21 | 0.10 |  |
| N,N-dimethyl formamide | 35.70 | 2.44 |  |
| 2,2-dimethyl-1-pentanol | 12.08 | 1.43 |  |
| dimethyl phthalate | 49.20 | 6.63 |  |
| dimethyl sulfone | 37.76 | 2.59 |  |
| dimethyl sulfoxide | 31.85 | 1.99 |  |
| N,N-dimethyltoluamine | 0.00 | 0.00 |  |
| di-n-propylamine | 35.25 | 3.74 |  |
| dioctyl phthalate | 23.64 | 7.60 |  |
| 1,4-dioxane | 100.00 | 6.92 |  |
| dipropylene glycol | 48.95 | 5.80 |  |
| dipropylene glycol methyl ether | 25.27 | 3.34 |  |
| dipropyl ether | 5.81 | 0.60 |  |
| dodecane | 0.00 | 0.00 |  |
| eicosane | 0.00 | 0.00 |  |
| ethane | 0.00 | 0.00 |  |
| ethanethiol | 0.00 | 0.00 |  |
| ethanol | 28.83 | 1.43 |  |
| ethanolamine | 100.00 | 5.97 |  |
| 2-ethoxyethyl acetate | 50.34 | 5.71 |  |
| ethyl acetate | 28.54 | 2.25 |  |
| ethylbenzene | 0.00 | 0.00 |  |
| ethylbenzoate | 22.49 | 2.49 |  |
| 2-ethyl-1-butanol | 13.90 | 1.43 |  |
| ethyl cinnamate | 25.05 | 3.31 |  |
| ethylene | 0.00 | 0.00 |  |
| ethylene carbonate | 100.00 | 5.95 |  |
| ethylenediamine | 100.00 | 6.25 |  |
| ethylene glycol | 100.00 | 5.72 |  |
| ethylene glycol butyl ether | 44.10 | 4.89 |  |
| ethylene glycol ethyl ether | 58.05 | 4.89 |  |
| ethylene glycol methyl ether | 68.95 | 4.89 |  |
| ethyl formate | 46.17 | 3.03 |  |
| ethyl heptanoate | 15.47 | 2.25 |  |
| ethyl hydrocinnamate | 19.30 | 2.65 |  |
| ethylphenyl ether | 0.00 | 0.00 |  |
| ethyl phenyl ketone | 14.39 | 1.48 |  |
| ethyl propionate | 24.42 | 2.25 |  |
| fluorobenzene | 0.00 | 0.00 |  |
| fluoromethane | 0.00 | 0.00 |  |
| p-fluorotoluene | 0.00 | 0.00 |  |
| formaldehyde | 100.00 | 3.11 |  |
| formamide | 100.00 | 4.02 |  |
| formic acid | 100.00 | 3.86 |  |
| furan | 10.47 | 0.52 |  |
| furfural | 78.23 | 5.09 |  |
| glycerol | 100.00 | 7.82 |  |
| n-heptane | 0.00 | 0.00 |  |
| 1-heptanol | 16.67 | 1.94 |  |
| 2-heptanol | 12.30 | 1.43 |  |
| 3-heptanol | 12.30 | 1.43 |  |
| 4-heptanol | 12.30 | 1.43 |  |
| 2-heptanone | 10.05 | 1.11 |  |
| hexadecane | 0.00 | 0.00 |  |
| hexamethylphosphoramide | 100.00 | 15.28 |  |
| hexanal | 23.85 | 2.32 |  |
| n-hexane | 0.00 | 0.00 |  |
| 1-hexanol | 18.83 | 1.94 |  |
| 2-hexanol | 13.90 | 1.43 |  |
| 3-hexanol | 13.90 | 1.43 |  |
| 2-hexanone | 11.42 | 1.11 |  |
| hexylene glycol | 25.69 | 2.87 |  |
| hydrocinnamonitrile | 22.93 | 2.34 |  |
| iodobenzene | 0.00 | 0.00 |  |
| iodomethane | 0.00 | 0.00 |  |
| isoamyl acetate | 18.92 | 2.25 |  |
| isobutane | 0.00 | 0.00 |  |
| isobutyl acetate | 21.30 | 2.25 |  |
| isobutyl isobutyrate | 17.00 | 2.25 |  |
| isooctane | 0.00 | 0.00 |  |
| isophorone | 9.18 | 1.11 |  |
| isopropyl palmitate | 8.08 | 2.25 |  |
| lactic acid | 71.01 | 5.16 |  |
| mesitylene | 0.00 | 0.00 |  |
| methacrylic acid | 45.85 | 3.37 |  |
| methane | 0.00 | 0.00 |  |
| methanol | 39.40 | 1.43 |  |
| o-methoxyphenol | 15.11 | 1.37 |  |
| methyl acetate | 34.39 | 2.25 |  |
| methylal | 38.23 | 2.73 |  |
| 2-methylbutane | 0.00 | 0.00 |  |
| 2-methyl-1-butanol | 15.97 | 1.43 |  |
| 2-methyl-2-butanol | 15.78 | 1.43 |  |
| 3-methyl-1-butanol | 15.95 | 1.43 |  |
| methylcyclohexane | 0.00 | 0.00 |  |
| methylene chloride | 0.00 | 0.00 |  |
| methyl formate | 57.92 | 3.03 |  |
| 2-methyl-2-heptanol | 10.94 | 1.43 |  |
| 3-methyl-3-heptanol | 10.94 | 1.43 |  |
| 3-methylhexane | 0.00 | 0.00 |  |
| methyl hexanoate | 18.94 | 2.25 |  |
| 2-methyl-2-hexanol | 12.18 | 1.43 |  |
| 3-methyl-3-hexanol | 12.18 | 1.43 |  |
| 5-methyl-2-hexanol | 12.29 | 1.43 |  |
| methyl isoamyl ketone | 10.03 | 1.11 |  |
| methyl isobutyl ketone | 11.40 | 1.11 |  |
| methyl methacrylate | 25.81 | 2.25 |  |
| 1-methylnaphthalene | 0.00 | 0.00 |  |
| methyl octanoate | 15.47 | 2.25 |  |
| methyl oleate | 8.22 | 2.25 |  |
| 3-methylpentane | 0.00 | 0.00 |  |
| 2-methyl-1-pentanol | 13.90 | 1.43 |  |
| 2-methyl-2-pentanol | 13.74 | 1.43 |  |
| 2-methyl-3-pentanol | 13.89 | 1.43 |  |
| 3-methyl-2-pentanol | 13.88 | 1.43 |  |
| 3-methyl-3-pentanol | 13.74 | 1.43 |  |
| 4-methyl-1-pentanol | 18.80 | 1.94 |  |
| 4-methyl-2-pentanol | 13.88 | 1.43 |  |
| methyl phenoxyacetate | 35.60 | 4.22 |  |
| methyl phenylacetate | 24.20 | 2.70 |  |
| 1-methylpiperidine | 1.09 | 0.10 |  |
| 2-methyl-1-propanol | 18.73 | 1.43 |  |
| 2-methyl-2-propanol | 18.46 | 1.43 |  |
| methyl propionate | 28.54 | 2.25 |  |
| N-methylpyrrole | 0.00 | 0.00 |  |
| N-methyl pyrrolidinone | 38.03 | 3.03 |  |
| morpholine | 100.00 | 7.23 |  |
| naphthalene | 0.00 | 0.00 |  |
| nitrobenzene | 30.75 | 2.47 |  |
| 2-nitrobenzonitrile | 49.42 | 4.75 |  |
| nitroethane | 42.93 | 2.65 |  |
| nitromethane | 54.74 | 2.65 |  |
| 2-nitropropane | 35.29 | 2.65 |  |
| n-nonane | 0.00 | 0.00 |  |
| 1-nonanol | 13.57 | 1.94 |  |
| nonoxynol-1 | 17.20 | 3.91 |  |
| nonylphenol | 4.12 | 0.80 |  |
| n-octane | 0.00 | 0.00 |  |
| octanoic acid | 33.44 | 4.40 |  |
| 1-octanol | 14.96 | 1.94 |  |
| 2-octanol | 11.04 | 1.43 |  |
| 2-octanone | 8.97 | 1.11 |  |
| n-octyl pyrrolidinone | 36.21 | 6.24 |  |
| oleic acid | 16.94 | 4.40 |  |
| pentachloroethane | 0.00 | 0.00 |  |
| n-pentane | 0.00 | 0.00 |  |
| 1-pentanol | 21.63 | 1.94 |  |
| 2-pentanol | 15.96 | 1.43 |  |
| 3-pentanol | 15.96 | 1.43 |  |
| 2-pentanone | 13.23 | 1.11 |  |
| 3-pentanone | 13.23 | 1.11 |  |
| perfluoroheptane | 61.30 | 10.60 |  |
| phenethyl acetate | 22.22 | 2.76 |  |
| phenol | 11.66 | 0.79 |  |
| PhenylC(COOEt)2H | 31.09 | 5.44 |  |
| Phenyl(CH2)3C(=O)Me | 8.62 | 1.11 |  |
| Phenyl(CH2)3OC(=O)Me | 20.07 | 2.76 |  |
| Phenyl(CH2)3OMe | 4.92 | 0.60 |  |
| PhenylOCH2C(=O)N(Me)2 | 26.86 | 3.60 |  |
| phenylpropyl ether | 0.00 | 0.00 |  |
| a-pinene | 0.00 | 0.00 |  |
| piperazine | 100.00 | 7.49 |  |
| piperidine | 58.93 | 4.55 |  |
| n-propane | 0.00 | 0.00 |  |
| 1-propanol | 30.78 | 1.94 |  |
| 2-propanol | 22.70 | 1.43 |  |
| propenal | 43.84 | 2.31 |  |
| propene | 0.00 | 0.00 |  |
| 1-propenol | 37.29 | 2.17 |  |
| propionitrile | 20.67 | 1.18 |  |
| propyl acetate | 24.42 | 2.25 |  |
| n-propylamine | 53.73 | 3.52 |  |
| propylene glycol | 63.63 | 4.49 |  |
| propyne | 0.00 | 0.00 |  |
| pyridine | 8.37 | 0.49 |  |
| 2-pyrrolidinone | 100.00 | 6.58 |  |
| 1,1,2,2-tetrabromoethane | 0.00 | 0.00 |  |
| 1,1,2,2-tetrachloroethane | 0.00 | 0.00 |  |
| tetrachloroethylene | 0.00 | 0.00 |  |
| tetrahydrofuran | 27.10 | 1.68 |  |
| tetrahydropyran | 8.08 | 0.60 |  |
| tetralin | 0.00 | 0.00 |  |
| 1,1,3,3-tetramethylurea | 17.26 | 1.83 |  |
| toluene | 0.00 | 0.00 |  |
| tributyl phosphate | 34.55 | 7.45 |  |
| 1,1,1-trichloroethane | 0.00 | 0.00 |  |
| trichloroethylene | 0.00 | 0.00 |  |
| trichlorofluoromethane | 0.00 | 0.00 |  |
| 1,1,2-trichlorotriflouroethane | 0.00 | 0.00 |  |
| triethylamine | 1.30 | 0.14 |  |
| triethylene glycol | 100.00 | 12.64 |  |
| triethylene glycol dimethyl ether | 59.36 | 9.13 |  |
| triethylene glycol oleyl ether | 34.05 | 12.31 |  |
| triethyl phosphate | 36.15 | 5.10 |  |
| trifluoromethylbenzene | 0.00 | 0.00 |  |
| tripropylamine | 1.19 | 0.18 |  |
| 2-Undecanol | 8.44 | 1.43 |  |
| vinyl acetate | 46.49 | 3.45 |  |
| vinyl chloride | 0.00 | 0.00 |  |
| water | 100.00 | 2.19 |  |
| m-xylene | 0.00 | 0.00 |  |
| o-xylene | 0.00 | 0.00 |  |
| p-xylene | 0.00 | 0.00 |  |

| Table 2-4. SURFACTANTS |  |  |
| --- | --- | --- |
| Name | Percent Hydrophilic surface area | Hydrophilic surface area cm2/mol 109 |
| ammonium cumenesulfonate | 49.46 | 7.35 |
| ammonium dodecylbenzenesulfonate | 27.42 | 7.37 |
| ammonium isostearate | 20.99 | 5.88 |
| ammonium laureth sulfate | 55.49 | 20.43 |
| ammonium laureth-5 sulfate | 59.31 | 23.89 |
| ammonium laureth-7 sulfate | 65.28 | 30.81 |
| ammonium laureth-12 sulfate | 74.59 | 48.10 |
| ammonium lauryl sulfate | 32.79 | 7.52 |
| ammonium myristyl sulfate | 29.33 | 7.52 |
| ammonium nonoxynol-4 sulfate | 46.15 | 17.43 |
| ammonium oleate | 21.35 | 5.88 |
| ammonium stearate | 21.00 | 5.89 |
| ammonium xylenesulfonate | 50.90 | 6.92 |
| arachidic acid | 15.12 | 4.42 |
| arachidonic acid | 16.16 | 4.42 |
| arachidyl propionate | 8.35 | 2.78 |
| ascorbic acid | 100.00 | 11.63 |
| ascorbyl dipalmitate | 24.83 | 13.60 |
| ascorbyl dipalmitate | 24.83 | 13.60 |
| ascorbyl palmitate | 37.33 | 12.36 |
| ascorbyl stearate | 34.61 | 12.46 |
| batyl alcohol | 23.53 | 7.47 |
| batyl isostearate | 14.85 | 8.30 |
| batyl stearate | 14.84 | 8.34 |
| behenamidopropyl dimethylamine | 18.02 | 7.22 |
| beheneth-5 | 39.28 | 19.25 |
| beheneth-10 | 55.13 | 36.56 |
| beheneth-20 | 70.52 | 71.16 |
| beheneth-30 | 78.04 | 105.77 |
| behenic acid | 13.85 | 4.42 |
| behenyl alcohol | 6.13 | 1.94 |
| behenyl imidazoline | 22.47 | 8.35 |
| benzyloctane | 0.00 | 0.00 |
| BHT | 2.93 | 0.57 |
| butyl acetyl ricinoleate | 14.43 | 5.06 |
| butyl myristate | 10.46 | 2.78 |
| butyl oleate | 8.84 | 2.78 |
| butyl phthalyl butyl glycolate | 45.07 | 11.39 |
| butyl stearate | 8.70 | 2.78 |
| calcium disodium EDTA | 100.00 | 26.18 |
| calcium pantothenate | 63.99 | 23.08 |
| calcium stearate | 18.27 | 9.90 |
| calcium stearoyl lactate | 25.49 | 18.92 |
| camphor | 8.97 | 1.11 |
| capramide DEA | 46.37 | 10.71 |
| capric acid | 27.85 | 4.42 |
| caproamphocarboxypropionate | 70.94 | 27.90 |
| caproamphoglycinate | 61.99 | 18.75 |
| caproamphopropionate | 63.49 | 20.00 |
| caproamphopropylsulfonate | 66.46 | 23.35 |
| caproic acid | 41.99 | 4.42 |
| caprylic acid | 33.50 | 4.42 |
| capryloamphocarboxypropionate | 75.61 | 27.81 |
| capryloamphoglycinate | 68.06 | 18.72 |
| capryloamphopropionate | 69.21 | 19.83 |
| capryloamphopropylsulfonate | 72.34 | 23.90 |
| carvone | 8.86 | 1.11 |
| cetacene | 22.43 | 7.39 |
| ceteth-1 | 19.93 | 5.41 |
| ceteth-2 | 28.99 | 8.87 |
| ceteth-4 | 42.08 | 15.79 |
| ceteth-5 | 46.98 | 19.25 |
| ceteth-6 | 51.10 | 22.71 |
| ceteth-10 | 62.71 | 36.55 |
| ceteth-12 | 66.67 | 43.48 |
| ceteth-16 | 72.51 | 57.32 |
| ceteth-20 | 76.60 | 71.16 |
| ceteth-25 | 80.28 | 88.46 |
| ceteth-30 | 82.95 | 105.76 |
| ceteth-45 | 87.88 | 157.67 |
| cetyl alcohol | 8.20 | 1.94 |
| cetyl lactate | 17.04 | 4.89 |
| cetyl myristate | 6.53 | 2.78 |
| cetyl octanoate | 8.09 | 2.80 |
| cetyl palmitate | 6.14 | 2.78 |
| cetyl phosphate | 24.10 | 6.61 |
| cholesterol | 4.47 | 1.44 |
| cocamide | 24.54 | 4.60 |
| cocamide DEA | 41.91 | 10.86 |
| cocamide MEA | 36.54 | 8.14 |
| cocamine oxide | 37.09 | 8.33 |
| cocodiaminepropane | 34.64 | 8.21 |
| deceth-6 | 62.35 | 22.71 |
| decyl alcohol | 12.40 | 1.94 |
| decyl betaine | 49.86 | 11.39 |
| decyl oleate | 7.00 | 2.75 |
| diamyl sodium sulfosuccinate | 50.74 | 14.48 |
| dicocamine | 10.84 | 3.76 |
| diisobutyl sodium sulfosuccinate | 52.09 | 13.34 |
| dimethyl hexanediol | 20.68 | 2.87 |
| dimethyl hexynediol | 21.55 | 2.88 |
| dimethyl octynediol | 18.09 | 2.87 |
| dimethyl oleamine | 0.45 | 0.13 |
| dioctyl sodium sulfosuccinate | 39.70 | 14.37 |
| disodium deceth-6 sulfosuccinate | 73.71 | 38.33 |
| disodium isodecyl sulfosuccinate | 56.35 | 17.70 |
| disodium nonoxynol-10 sulfosuccinate | 70.94 | 49.00 |
| disodiumoleamidoPEG-2 sulfosuccinate | 55.86 | 27.43 |
| disodium stearyl sulfosuccinate | 45.59 | 19.28 |
| ditridecyl sodium sulfosuccinate | 28.92 | 14.31 |
| dodecylbenzenesulfonic acid | 23.16 | 5.90 |
| dodecylbenzyltrimonium chloride | 64.05 | 5.28 |
| ethyl citroate | 50.62 | 12.36 |
| ethyl dicocamine | 0.38 | 0.14 |
| 2-ethylhexylsulfate | 37.43 | 6.05 |
| ethyl tributylcitroate | 40.91 | 13.18 |
| ethyl trioctylcitroate | 27.46 | 13.19 |
| glycerin | 100.00 | 7.83 |
| glyceryl distearate | 16.87 | 9.51 |
| glyceryl laurate | 36.07 | 8.59 |
| glyceryl linoleate | 27.72 | 8.62 |
| glyceryl myristate | 32.40 | 8.65 |
| glyceryl oleate | 27.29 | 8.62 |
| glyceryl stearate | 27.02 | 8.65 |
| glycol distearate | 13.57 | 7.37 |
| glycol stearate | 21.91 | 6.57 |
| hexylene glycol | 25.62 | 2.87 |
| isopropyl myristate | 8.95 | 2.26 |
| isopropyl palmitate | 8.09 | 2.26 |
| isopropyl stearate | 7.39 | 2.26 |
| lauramide DEA | 41.34 | 10.61 |
| lauramide MEA | 36.32 | 8.06 |
| lauramide MIPA | 30.98 | 7.29 |
| lauramidopropylamine oxide | 48.88 | 13.52 |
| lauramidopropyl betaine | 54.02 | 16.61 |
| lauramidopropyl dimethylamine | 26.81 | 7.12 |
| lauramine | 23.90 | 4.44 |
| lauramine oxide | 37.05 | 8.32 |
| laureth-3 | 42.92 | 12.32 |
| laureth-4 | 49.08 | 15.79 |
| laureth-7 | 61.49 | 26.17 |
| laureth-9 | 66.87 | 33.09 |
| laureth-15 | 76.66 | 53.83 |
| laureth-23 | 83.25 | 81.49 |
| lauroamphocarboxyglycinate | 61.38 | 22.41 |
| lauroamphocarboxypropinate | 66.22 | 27.65 |
| lauroamphoglycinate | 56.47 | 18.30 |
| lauroamphopropylsulfonate | 61.82 | 22.81 |
| lauroyl sarcosine | 29.27 | 6.95 |
| laurtrimonium chloride | 70.76 | 6.13 |
| lauryl alcohol | 10.59 | 1.94 |
| lauryl betaine | 46.16 | 12.01 |
| lauryl lactate | 20.83 | 4.83 |
| linoleamide DEA | 32.50 | 10.64 |
| linoleamide MEA | 27.60 | 8.07 |
| linoleamide MIPA | 23.87 | 7.30 |
| linoleic acid | 17.25 | 4.41 |
| linolenic acid | 17.57 | 4.41 |
| magnesium laureth-4 sulfate | 54.36 | 38.94 |
| magnesium lauryl sulfate | 31.71 | 13.94 |
| MEA-lauryl sulfate | 64.13 | 6.00 |
| meroxapol 105 | 65.40 | 95.18 |
| meroxapol 171 | 34.59 | 45.41 |
| meroxapol 172 | 41.27 | 61.32 |
| meroxapol 174 | 55.71 | 109.73 |
| meroxapol 251 | 33.52 | 65.63 |
| meroxapol 252 | 41.75 | 93.30 |
| meroxapol 255 | 62.57 | 219.85 |
| meroxapol 254 | 55.05 | 161.07 |
| meroxapol 311 | 32.61 | 74.36 |
| meroxapol 312 | 39.90 | 102.02 |
| meroxapol 314 | 54.63 | 185.01 |
| methyl caproate | 19.08 | 2.27 |
| methyl caprylate | 15.59 | 2.27 |
| methyl laurate | 11.42 | 2.27 |
| methyl linoleate | 8.43 | 2.27 |
| methyl myristate | 10.07 | 2.27 |
| methyl oleate | 8.28 | 2.27 |
| methyl palmitate | 9.00 | 2.27 |
| methyl ricinoleate | 13.13 | 3.70 |
| methyl stearate | 8.14 | 2.27 |
| MIPA-laureth-4 sulfate | 62.41 | 12.07 |
| MIPA-lauryl sulfate | 49.70 | 5.61 |
| myreth-3 | 39.31 | 12.31 |
| myreth-4 | 45.34 | 15.77 |
| myreth-3 caprate | 29.25 | 13.13 |
| myreth-3 myristate | 25.93 | 13.13 |
| myristalkonium chloride | 66.96 | 6.35 |
| myristamide DEA | 37.45 | 10.61 |
| myristamide MEA | 32.39 | 8.05 |
| myristamine oxide | 33.14 | 8.33 |
| myristic acid | 20.86 | 4.42 |
| myristic alcohol | 9.25 | 1.94 |
| myristyl lactate | 18.60 | 4.84 |
| myristyl myristate | 6.91 | 2.76 |
| nonoxynol-1 | 17.18 | 3.89 |
| nonoxynol-2 | 22.58 | 5.89 |
| nonoxynol-3 | 31.66 | 9.38 |
| nonoxynol-4 | 38.79 | 12.79 |
| nonoxynol-5 | 44.58 | 16.24 |
| nonoxynol-6 | 49.38 | 19.69 |
| nonoxynol-7 | 53.42 | 23.15 |
| nonoxynol-8 | 56.88 | 26.63 |
| nonoxynol-9 | 59.85 | 30.09 |
| nonoxynol-10 | 62.43 | 33.55 |
| nonoxynol-11 | 64.71 | 37.01 |
| nonoxynol-12 | 66.72 | 40.47 |
| nonoxynol-13 | 68.51 | 43.92 |
| nonoxynol-14 | 70.12 | 47.38 |
| nonoxynol-15 | 71.58 | 50.83 |
| nonoxynol-20 | 77.14 | 68.12 |
| nonoxynol-30 | 83.57 | 102.70 |
| nonoxynol-40 | 87.18 | 137.27 |
| nonoxynol-50 | 89.49 | 171.85 |
| nonoxynol-6 carboxylate | 51.87 | 22.02 |
| nonoxynol-9 carboxylate | 61.32 | 32.39 |
| nonoxynol-9 phosphate | 62.63 | 33.83 |
| nonyl nonoxynol-10 phosphate | 52.78 | 37.21 |
| nonyl nonoxynol-7 phosphate | 45.10 | 26.96 |
| octoxynol-1 | 18.16 | 3.88 |
| octoxynol-2 | 23.85 | 5.93 |
| octoxynol-3 | 33.14 | 9.38 |
| octoxynol-4 | 40.42 | 12.84 |
| octoxynol-5 | 46.27 | 16.30 |
| octoxynol-7 | 55.09 | 23.21 |
| octoxynol-9 | 61.42 | 30.13 |
| octoxynol-10 | 63.96 | 33.59 |
| octoxynol-13 | 69.91 | 43.96 |
| octoxynol-16 | 74.17 | 54.33 |
| octoxynol-30 | 84.45 | 102.74 |
| octoxynol-40 | 87.89 | 137.32 |
| octyl isononanoate | 10.84 | 2.76 |
| octyl palmitate | 7.99 | 2.76 |
| octyl pelargonate | 10.94 | 2.76 |
| octyl stearate | 7.42 | 2.76 |
| oleamide | 17.56 | 4.62 |
| oleamide betaine | 43.59 | 16.72 |
| oleamide DEA | 32.00 | 10.71 |
| oleamide dimethylamine | 21.02 | 7.16 |
| oleamide MEA | 27.45 | 8.18 |
| oleamide MIPA | 23.80 | 7.41 |
| oleamine | 17.03 | 4.43 |
| oleamine oxide | 27.80 | 8.32 |
| oleic acid | 16.94 | 4.40 |
| oleoamphoglycinate | 45.90 | 18.32 |
| oleoamphopropionate | 47.89 | 19.88 |
| oleoamphopropylsulfonate | 51.45 | 22.93 |
| oleoyl sarcosine | 22.45 | 7.06 |
| oleth-2 | 27.09 | 8.85 |
| oleth-3 | 34.06 | 12.30 |
| oleth-4 | 39.80 | 15.75 |
| oleth-5 | 44.63 | 19.19 |
| oleth-6 | 48.73 | 22.63 |
| oleth-10 | 60.47 | 36.43 |
| oleth-12 | 64.54 | 43.34 |
| oleth-15 | 69.27 | 53.67 |
| oleth-20 | 74.87 | 70.94 |
| oleth-25 | 78.74 | 88.22 |
| oleth-10 phosphate | 62.78 | 40.17 |
| oleth-3 phosphate | 40.25 | 16.04 |
| oleth-4 phosphate | 45.01 | 19.49 |
| oleyl alcohol | 7.55 | 1.95 |
| oleyl betaine | 34.67 | 11.47 |
| oleyl oleate | 5.58 | 2.77 |
| palmitamide DEA | 34.32 | 10.63 |
| palmitamide MEA | 29.34 | 8.07 |
| palmitamine | 18.55 | 4.42 |
| palmitamine oxide | 30.03 | 8.33 |
| palmitic acid | 18.49 | 4.40 |
| panthenol | 70.27 | 12.09 |
| pareth-15-3 | 41.09 | 12.30 |
| pareth-15-5 | 52.14 | 19.21 |
| pareth-15-7 | 59.71 | 26.12 |
| pareth-15-9 | 65.21 | 33.04 |
| pareth-15-12 | 71.08 | 43.30 |
| pareth-15-15 | 75.28 | 53.67 |
| pareth-15-20 | 80.11 | 70.96 |
| pareth-15-30 | 85.69 | 105.54 |
| pareth-15-40 | 88.83 | 140.11 |
| PEG-4 | 100.00 | 16.09 |
| PEG-6 | 100.00 | 23.01 |
| PEG-8 | 100.00 | 29.92 |
| PEG-12 | 100.00 | 43.76 |
| PEG-20 | 100.00 | 71.42 |
| PEG-32 | 100.00 | 112.91 |
| PEG-75 | 100.00 | 261.59 |
| PEG-150 | 100.00 | 520.92 |
| PEG-5 castor oil | 32.34 | 31.94 |
| PEG-15 castor oil | 49.89 | 66.51 |
| PEG-16 castor oil | 51.16 | 69.97 |
| PEG-20 castor oil | 55.65 | 83.82 |
| PEG-25 castor oil | 60.21 | 101.09 |
| PEG-30 castor oil | 62.93 | 116.52 |
| PEG-36 castor oil | 66.66 | 137.26 |
| PEG-40 castor oil | 68.11 | 150.58 |
| PEG-3 cocamide | 51.48 | 15.00 |
| PEG-6 cocamide | 64.22 | 25.37 |
| PEG-2 cocamine | 39.19 | 9.97 |
| PEG-5 cocamine | 54.26 | 19.45 |
| PEG-10 cocamine | 69.14 | 36.74 |
| PEG-15 cocamine | 76.72 | 54.03 |
| PEG-15 cocoate | 78.09 | 54.95 |
| PEG-5 cocoate | 56.92 | 20.37 |
| PEG-4 dilaurate | 36.51 | 17.73 |
| PEG-6 dilaurate | 44.43 | 24.64 |
| PEG-8 dilaurate | 50.59 | 31.56 |
| PEG-8 dioleate | 40.74 | 31.56 |
| PEG-8 distearate | 40.27 | 31.56 |
| PEG-12 distearate | 49.23 | 45.39 |
| PEG-150 distearate | 91.19 | 480.36 |
| PEG-30 glyceryl cocoate | 87.95 | 112.49 |
| PEG-20 glyceryl laurate | 83.46 | 77.79 |
| PEG-30 glyceryl laurate | 87.94 | 112.37 |
| PEG-20 glyceryl oleate | 77.20 | 77.74 |
| PEG-30 glyceryl oleate | 83.04 | 112.37 |
| PEG-20 glyceryl stearate | 76.91 | 77.47 |
| PEG-30 glyceryl stearate | 82.76 | 112.37 |
| PEG-2 laurate | 39.34 | 10.00 |
| PEG-4 laurate | 52.32 | 16.91 |
| PEG-8 laurate | 66.61 | 30.74 |
| PEG-12 laurate | 74.31 | 44.57 |
| PEG-2 oleamine | 30.47 | 10.05 |
| PEG-5 oleamine | 45.01 | 19.53 |
| PEG-15 oleamine | 68.35 | 51.50 |
| PEG-10 oleate | 62.16 | 37.66 |
| PEG-2 oleate | 30.37 | 10.00 |
| PEG-4 oleate | 42.46 | 16.91 |
| PEG-5 oleate | 47.05 | 20.37 |
| PEG-6 oleate | 50.97 | 23.83 |
| PEG-8 oleate | 57.29 | 30.74 |
| PEG-9 oleate | 59.91 | 34.08 |
| PEG-12 oleate | 66.04 | 44.57 |
| PEG-14 oleate | 69.20 | 51.27 |
| PEG-20 oleate | 75.91 | 72.23 |
| PEG-25 propylene glycol stearate | 76.70 | 84.90 |
| PEG-2 stearamine | 30.08 | 10.07 |
| PEG-5 stearamine | 44.54 | 19.54 |
| PEG-10 stearamine | 60.21 | 36.83 |
| PEG-15 stearamine | 68.98 | 54.12 |
| PEG-50 stearamine | 87.80 | 175.14 |
| PEG-2 stearate | 29.91 | 10.01 |
| PEG-4 stearate | 41.91 | 16.93 |
| PEG-5 stearate | 46.49 | 20.39 |
| PEG-6 stearate | 50.40 | 23.84 |
| PEG-8 stearate | 56.72 | 30.76 |
| PEG-10 stearate | 61.62 | 37.67 |
| PEG-12 stearate | 65.52 | 44.59 |
| PEG-20 stearate | 75.48 | 72.25 |
| PEG-32 stearate | 82.90 | 113.74 |
| PEG-40 stearate | 85.77 | 141.41 |
| PEG-50 stearate | 88.23 | 175.98 |
| PEG-75 stearate | 91.79 | 262.43 |
| PEG-100 stearate | 93.70 | 348.87 |
| PEG-150 stearate | 95.70 | 521.75 |
| PEG-2 stearmonium chloride | 64.47 | 6.36 |
| PEG-15 stearmonium chloride | 86.31 | 30.40 |
| pelargonic acid | 30.37 | 4.40 |
| pentaerythritol tetrastearate | 13.59 | 14.72 |
| phenethyl alcohol | 20.43 | 1.94 |
| phenoxydiglycol | 43.11 | 5.92 |
| phenoxyethanol | 37.87 | 3.89 |
| poloxamer 101 | 36.57 | 33.80 |
| poloxamer 105 | 61.94 | 96.07 |
| poloxamer 108 | 84.28 | 332.41 |
| poloxamer 122 | 43.69 | 60.13 |
| poloxamer 123 | 48.83 | 73.96 |
| poloxamer 124 | 56.73 | 101.62 |
| poloxamer 181 | 33.70 | 56.28 |
| poloxamer 182 | 43.25 | 86.52 |
| poloxamer 183 | 48.77 | 104.63 |
| poloxamer 184 | 51.61 | 121.07 |
| poloxamer 185 | 60.12 | 166.92 |
| poloxamer 188 | 83.35 | 554.19 |
| poloxamer 212 | 42.74 | 96.40 |
| poloxamer 215 | 61.58 | 207.04 |
| poloxamer 217 | 75.62 | 400.67 |
| poloxamer 231 | 37.67 | 87.00 |
| poloxamer 234 | 56.86 | 193.18 |
| poloxamer 235 | 60.82 | 228.05 |
| poloxamer 237 | 76.72 | 474.26 |
| poloxamer 238 | 83.27 | 716.30 |
| poloxamer 282 | 41.60 | 123.53 |
| poloxamer 284 | 53.51 | 199.60 |
| poloxamer 288 | 83.81 | 898.06 |
| poloxamer 331 | 35.68 | 110.55 |
| poloxamer 334 | 58.12 | 276.52 |
| poloxamer 335 | 61.98 | 324.93 |
| poloxamer 338 | 82.62 | 947.31 |
| poloxamer 401 | 32.32 | 118.05 |
| poloxamer 402 | 40.23 | 166.46 |
| poloxamer 403 | 46.90 | 215.77 |
| poloxamer 407 | 75.31 | 754.27 |
| poloxamine 304 | 52.79 | 65.77 |
| poloxamine 504 | 53.98 | 138.56 |
| poloxamine 701 | 33.01 | 87.19 |
| poloxamine 702 | 38.34 | 119.32 |
| poloxamine 704 | 53.15 | 234.40 |
| poloxamine 707 | 72.54 | 740.70 |
| polyacrylamide | 99.59 | 517.39 |
| polyacrylic acid | 99.49 | 501.13 |
| polysorbate 20 | 84.13 | 81.75 |
| polysorbate 21 | 63.41 | 26.72 |
| polysorbate 40 | 84.18 | 82.04 |
| polysorbate 60 | 77.81 | 82.04 |
| polysorbate 61 | 53.19 | 26.61 |
| polysorbate 65 | 54.26 | 83.54 |
| polysorbate 80 | 78.07 | 81.64 |
| polysorbate 81 | 56.83 | 30.22 |
| polysorbate 85 | 54.79 | 83.33 |
| polyvinyl acetate | 52.17 | 334.99 |
| polyvinyl alcohol | 72.57 | 252.45 |
| potassium dodecylbenzenesulfonate | 35.45 | 10.51 |
| potassium laurate | 39.17 | 9.07 |
| potassium myristate | 35.13 | 9.07 |
| potassium oleate | 29.56 | 9.07 |
| potassium palmitate | 31.85 | 9.07 |
| potassium ricinoleate | 33.40 | 10.51 |
| potassium sorbate | 62.22 | 8.82 |
| potassium stearate | 29.13 | 9.07 |
| PPG-9 | 29.84 | 13.56 |
| PPG-12 | 30.53 | 18.27 |
| PPG-17 | 26.76 | 22.44 |
| PPG-20 | 27.63 | 27.15 |
| PPG-26 | 25.52 | 32.42 |
| PPG-30 | 25.20 | 36.85 |
| PPG-24-buteth-27 | 56.16 | 121.40 |
| PPG-26-buteth-26 | 54.05 | 120.16 |
| PPG-28-buteth-35 | 58.36 | 153.50 |
| PPG-33-buteth-45 | 60.21 | 193.62 |
| PPG-14 butyl ether | 24.16 | 18.08 |
| PPG-40 butyl ether | 23.50 | 46.91 |
| PPG-25 diethylmonium chloride | 64.03 | 19.40 |
| PPG-9 diethylmonium chloride | 66.57 | 9.97 |
| PPG-40 diethylmonium chloride | 64.21 | 29.97 |
| PPG-27 glyceryl ether | 28.30 | 38.88 |
| PPG-55 glyceryl ether | 25.54 | 69.42 |
| PPG-2 methyl ether | 36.06 | 4.77 |
| PPG-3 methyl ether | 32.61 | 5.88 |
| PPG-3-myreth-3 | 33.10 | 15.13 |
| PPG-3 myristyl ether | 16.64 | 5.88 |
| PPG-36 oleate | 21.39 | 42.47 |
| PPG-9-steareth-3 | 27.29 | 21.78 |
| PPG-11 stearyl ether | 18.65 | 14.74 |
| PPG-15 stearyl ether | 21.13 | 20.76 |
| propylene glycol | 63.63 | 4.49 |
| propylene glycol dipelargonate | 19.44 | 6.13 |
| propylene glycol laurate | 22.80 | 5.31 |
| propylene glycol stearate | 16.98 | 5.31 |
| PVP | 76.06 | 600.66 |
| quaternium-14 | 64.37 | 5.56 |
| quaternium-15 | 92.53 | 7.46 |
| quaternium-22 | 100.00 | 14.44 |
| quaternium-26 | 66.61 | 6.27 |
| quaternium-30 | 66.61 | 6.27 |
| quaternium-45 | 82.62 | 6.92 |
| quaternium-51 | 81.68 | 7.76 |
| quaternium-52 | 82.86 | 22.78 |
| ricinoleamide DEA | 35.54 | 12.04 |
| ricinoleamide MEA | 31.18 | 9.48 |
| ricinoleamide MIPA | 27.40 | 8.71 |
| ricinoleamidopropyl betaine | 46.52 | 18.24 |
| ricinoleamidopropyl dimethylamine | 24.53 | 8.54 |
| ricinoleic acid | 21.77 | 5.83 |
| sodium benzoate | 69.40 | 7.84 |
| sodium cetyl sulfate | 31.58 | 9.39 |
| sodium cocoate | 34.25 | 7.34 |
| sodium cocoyl isethionate | 43.85 | 12.00 |
| sodium cocoyl isethionate | 43.84 | 12.03 |
| sodium C14-16 olefin sulfonate | 31.45 | 8.72 |
| sodium cumenesulfonate | 56.96 | 9.41 |
| sodium dodecylbenzenesulfonate | 32.64 | 9.27 |
| sodium glyceryl oleate phosphate | 39.65 | 15.05 |
| sodium laureth-4 sulfate | 57.25 | 21.89 |
| sodium lauroyl sarcosinate | 37.05 | 9.88 |
| sodium lauryl sulfate | 38.63 | 9.38 |
| sodium methyl cocoyl taurate | 39.26 | 11.38 |
| sodium methyl cocoyl taurate | 39.26 | 11.38 |
| sodium methyl oleoyl taurate | 31.19 | 11.38 |
| sodium myreth-4 sulfate | 53.52 | 21.89 |
| sodium myristyl sulfate | 34.69 | 9.39 |
| sodium nonoxynol-9 phosphate | 64.09 | 36.92 |
| sodium nonoxynol-4 sulfate | 47.93 | 18.81 |
| sodium octoxynol-2 ethane sulfonate | 43.87 | 14.90 |
| sodium octoxynol-2 ethane sulfonate | 43.87 | 14.90 |
| sodium octoxynol-2 ethane sulfonate | 43.95 | 14.84 |
| sodium octyl sulfate | 47.02 | 8.95 |
| sodium oleate | 25.37 | 7.34 |
| sodium oleth-7 phosphate | 57.92 | 32.82 |
| sodium pareth-25 sulfate | 83.27 | 94.37 |
| sodium stearate | 24.94 | 7.34 |
| sodium stearyl lactate | 29.96 | 11.81 |
| sodium trideceth-4 sulfate | 55.32 | 21.89 |
| sodium tridecyl carboxylate | 64.94 | 32.75 |
| sodium trioleate | 17.42 | 14.58 |
| sodium xylenesulfonate | 53.56 | 8.05 |
| sorbitan dioleate | 25.49 | 15.69 |
| sorbitan laurate | 45.30 | 12.78 |
| sorbitan oleate | 35.77 | 12.78 |
| sorbitan palmitate | 38.12 | 12.78 |
| sorbitan stearate | 35.32 | 12.78 |
| sorbitan trioleate | 17.42 | 14.47 |
| sorbitan tristearate | 17.09 | 14.46 |
| soyamine | 17.48 | 4.47 |
| stearalkonium chloride | 64.47 | 6.36 |
| stearamide | 17.12 | 4.56 |
| stearamide DEA | 31.56 | 10.61 |
| stearamide MEA | 26.70 | 8.05 |
| stearamide MEA-stearate | 16.30 | 8.86 |
| stearamidopropyl dimethylamine | 20.60 | 7.11 |
| stearamine | 16.87 | 4.48 |
| stearamine oxide | 27.45 | 8.35 |
| steareth-2 | 26.75 | 8.82 |
| steareth-4 | 39.37 | 15.69 |
| steareth-7 | 51.80 | 26.14 |
| steareth-20 | 74.51 | 71.09 |
| steareth-40 | 85.22 | 140.25 |
| stearic acid | 16.63 | 4.40 |
| stearyl alcohol | 7.37 | 1.94 |
| stearyl stearate | 5.46 | 2.76 |
| TEA-dodecylbenzenesulfonate | 61.58 | 9.42 |
| TEA-lauryl sulfate | 64.13 | 9.48 |
| TEA-oleamido PEG-2 sulfosuccinate | 74.84 | 17.13 |
| tetrasodium dicarboxyethyl stearyl sulfosuccinate | 58.70 | 33.27 |
| toluene sulfonic acid | 55.12 | 5.90 |
| trideceth-3 | 41.05 | 12.31 |
| trideceth-6 | 56.20 | 22.68 |
| trideceth-8 | 62.61 | 29.60 |
| trideceth-10 | 67.38 | 36.52 |
| trideceth-12 | 71.07 | 43.43 |
| trideceth-15 | 75.27 | 53.80 |
| trideceth-7 carboxylic acid | 59.85 | 26.35 |
| trideceth-6 phosphate | 59.92 | 26.43 |
| trilaurin | 18.17 | 10.27 |
| triolein | 12.98 | 10.27 |
| tristearin | 12.76 | 10.27 |
| undecylenamide DEA | 44.51 | 10.61 |
| undecylenamide MEA | 39.54 | 8.05 |